CCNA1 Module 10 exam picture descriptions

Refer to the exhibit. The small office LAN shown in the exhibit

Refer to the exhibit. Host A pings host B.

Diagram depicts a network consisting of 2 hosts, 2 hubs and 2 routers.

Host A (192.168.10.34/27 Mac BBBB.3333.5677 connects to a hub, which in turn connects to router R4 on port IP 192.168.10.33/27 Mac 9999.DADC.1234.

R4 (IP 192.168.10.65/27) connects to R5 (IP 192.168.10.66/27)

R5 (port IP 192.168.10.129/27, mac 5555.AAAA.6666) connects to the second hub. The second hub connects to host B (Ip 192.168.10.134/27 mac DDDD.4444.1357)

Refer to the exhibit. A student in the Cisco network class

Diagram depicts a network with 4 hosts a layer 2 switch and the internet (cloud). The 4 hosts are connected to the hub which is also conneceted to the layer 2 switch. The switch connects to the internet cloud.

Refer to the exhibit. A newly hired technician is testing

Diagram shows 5 hosts connected to a hub.

Refer to the exhibit. The network administrator has assigned

Diagram depicts a network consisting of 4 hosts, a server, a switch, a router and an ISP (cloud). The for hosts and the server are connected to the switch. The switch connects to the router (called LBMISS). The router connects to the ISP.

Host A is connected to the LAN,

Diagram depicts a host (A), switch1, router1 and an ISP. Host A has the IP address 198.18.166.65

Subnet mask 255.255.255.240

Gateway 198.18.166.33

Host a connects to the switch, which in tur connects to the router with IP address 198.18.166.33/27.

The router connects to the ISP via a serial connection, IP address 192.0.2.179/27.

Refer to the exhibit. A technician is planning an addressing scheme

Diagram depicts a workstation (A) connected to a router, which in turn connects to a server. Network configuration is below.

Host A

IP 192.168.200.66

Subnet mask 255.255.255.192

Default gateway 192.168.200.65

Router

Connection to host A IP address 192.168.200.65/26

Connection to server  IP address 192.168.200.126/26

Server IP address 192.168.200.129/26

Refer to the exhibit. After host 2 is connected

Diagram depicts 2 hosts (1 & 2 ) a switch, a router and an ISP (cloud)

Host 1 IP 192.168.2.34/27

Host 2 IP 192.168.3.35/27

Both connect to the switch

Switch connects to the router port (IP 192.168.2.33/27)

Router then connects to the ISP, no IP given.

Refer to the exhibit. A network administrator is planning the addressing scheme for the LAN using

Diagram depicts 25 hosts connected to a switch. The switch connects to the router, router connects to ISP.

End of diagrams