# Lab 10.3.5c Subnetting a Class B Network

Objective

The objective of this lab is to provide a subnetting scheme using a Class B network

Background / Preparation

This is a written lab and is to be performed without the aid of an electronic calculator. ABC Manufacturing has acquired a Class B address, 172.16.0.0. The company needs to create a subnetting scheme to provide the following:

• 36 subnets with at least 100 hosts

• 24 subnets with at least 255 hosts

• 10 subnets with at least 50 hosts

It is not necessary to supply an address for the WAN connection since it is supplied by the Internet service provider.

Step 1 Given this Class B network address and these requirements answer the following questions

How many subnets are needed for this network?

What is the minimum number of bits that can be borrowed?

What is the subnet mask for this network?

1. Dotted decimal

2. Binary

3. Slash format

How many usable subnetworks are there?

How many usable hosts are there per subnet?

Step 2 Complete the following for the first three subnets and the last 4 subnets (copy headings if you wish)

Subnetwork #

Subnetwork ID

Host Range

Broadcast ID

What is the host range for subnet two?

What is the broadcast address for the 126th subnet?

What is the broadcast address for the major network?