I.  Basic Router Configuration  - 8 Steps to Success:  IP Basics

The commands listed below are to assist you in setting up your router.  The commands are only examples and do not reflect the configuration of any actual network.  Your actual  commands, ip addresses, network addresses, passwords, etc., will depend upon your network  design.  Caution:  Some of the show commands may be done in user mode, while others may only be done in privileged mode.

Sample Network:  You may wish to configure the network below.

2 routers

Cabrillo

Ethernet IP 192.168.15.0/24

Serial0 DCE IP 10.16.1.0/24

Monterey

Ethernet 172.30.1.0/24

Serial 1 DTE IP .16.1.0/24

Step 1:  Physical Connections  Connect all of the interfaces including:

- Console:  Connect your PC/terminal to the console port via Terraterm (9600-8-N-1-no flow)

- Ethernet:  Connect Ethernet ports to a hub or a switch using a straight-through cable.  Use a crossover cable if going directly between Ethernet ports on two routers.

- Serial:  If going directly between two routers, don’t forget to connect one port via the DTE cable and  the other via the DCE cable.

Step 2:  Boot up the router  You may use the setup mode (setup dialogue) but this is to help you with configuring the router using the  Cisco IOS commands.  The setup mode will only allow you to configure the router with the basic features  and not with any advanced features.

Step 3:  Host Name and Passwords  It is a good idea to begin your configuration with the hostname and passwords.  This will remind you  what router you are configuring and it is also a good idea to add the security of passwords right away. Commands now follow, comments are in {}

Router(config)#  hostname  Cabrillo

Cabrillo(config)#  enable  secret  class

Cabrillo(config)#  line  vty  0  4

{If you are running EFS, you may increase the number of telnet sessions to more than 5.}

Cabrillo(config-line)#  login

Cabrillo(config-line)#  password cisco

Cabrillo(config-line)#  logging synchronous

{debug and other messages will not mess up screen when inputting commands.}

Cabrillo(config)#  line  con  0

Cabrillo(config-line)#  login

Cabrillo(config-line)#  password  cisco

Cabrillo(config-line)#  logging synchronous

{debug and other messages will not mess up screen when inputting commands.}

Step 4:  Adding IP Addresses

Next lets add the IP addresses, as this is a basic function of configuring routers.  Below is an example of  configuring both an Ethernet and Serial interface.  Don’t forget to use the proper subnet mask!  For Serial  interface with the DCE cable you will need to also add the clocking with the clockrate command.

Commands follow

Cabrillo(config)#  interface  ethernet 0

Cabrillo(config-if)#  ip address  192.168.15.1  255.255.255.0

Cabrillo(config-if)#  description  Engineering Network

Cabrillo(config-if)#  no  shutdown

Cabrillo(config)#  interface  serial  0

Cabrillo(config-if)#  ip address  10.16.1.0  255.255.255.0

Cabrillo(config-if)#  clock rate  56000    {DCE interface only!}  
Cabrillo(config-if)#  no  shutdown

Cabrillo(config-if)#  description  Network to ISP

Step 5a:  Adding Dynamic Routing:  RIP  If this router will be participating in a dynamic routing protocol like RIP or IGRP, you will need to enable  the routing protocol along with those directly connected networks that will be participating.  Only use the  classful network address, not the subnet address of the network!

Commands follow

Cabrillo(config)#  router  rip

Cabrillo(config-router)#  network  192.168.15.0   {NOT Subnet Address}

Cabrillo(config-router)#  network  10.0.0.0    {NOT Subnet Address}

Step 5b:  Adding Dynamic Routing:  IGRP  If this router will be participating in a dynamic routing protocol like RIP or IGRP, you will need to enable  the routing protocol along with those directly connected networks that will be participating.  Only use the  classful network address, not the subnet address of the network!

Cabrillo(config)#  router  igrp  10  {autonomous-system a.k.a. process-id}

Cabrillo(config-router)#  network  192.168.15.0   {NOT Subnet Address}

Cabrillo(config-router)#  network  10.0.0.0    {NOT Subnet Address}

Step 6:  Adding Default and Static Routes  If your router needs a default route (normally on a boundary router) or a static route to another network  (normally to a stub network which is not participating in the dynamic routing protocol of Step 5), then  you will need to configure these.

Commands follow

Cabrillo(config)#  ip  route  0.0.0.0  0.0.0.0  10.16.1.2    (ip-address-of-next-hop-router)

Cabrillo(config)#  router rip

Cabrillo(config-router)#  network 172.30.0.0

Cabrillo(config-router)#  default-information originate  {IOS 12.1 and  later you must use this command when configuring a default route   using a “quad-zero” static route.  Can use for RIP, but does not work with IGRP.}

Cabrillo(config)#  ip  default-network  10.0.0.0  {Adding a default route using a the ip default-network command.    Either one, not both of these commands.  Also need a default route at the router.  This is the only option with IGRP.  Works also with RIP.}

Cabrillo(config)#  ip  route  172.30.1.0  255.255.255.0  10.16.1.2  {Configuring a static route: network-address  subnet-mask  ip-address-of-next-hop-router}

Step 7:  Testing and Monitoring

At this point it is a good idea to start testing your network using various commands.

Example commands follow

Cabrillo#  show  ip  route

Cabrillo#  show  ip  interface  brief    {Great command!}

Cabrillo#  show  controller  s  0  {Shows whether or not the serial cable is DCE or DTE.}

Cabrillo#  ping  ip-address  {Extended ping, press return. }

Cabrillo#  traceroute  ip-address

Cabrillo#  debug  ip rip   {Remember to turn debug off when done, undebug  all}

Cabrillo#  terminal monitor  {If using debug from a telnet session, otherwise debug output will  go to the console.  Caution:  This will cause the debug output to go to all telnet sessions on this  router.}

Cabrillo#  terminal no monitor  {To un-do the command above.}

Be familiar with when you would use these and what they are showing you:

More example commands follow

Cabrillo#  show  ip  protocols

Cabrillo#  show  version

Cabrillo#  show  flash

Cabrillo#  show running config

Cabrillo#  show startup-config

Cabrillo#  show protocol

Cabrillo#  show ip protocols

Cabrillo#  show interface

Cabrillo#  show interface s 0

Cabrillo#  show interface e 0

Cabrillo#  show controller s 0

Cabrillo#  show controller e 0

Cabrillo#  show ip interface brief

Cabrillo#  show cdp interface

Cabrillo#  show cdp neighbor

Cabrillo#  show cdp neighbor detail

Cabrillo#  show ip route

Cabrillo#  show version

Cabrillo#  show flash

More Testing Commands  Here are some commands which may help you troubleshoot the router.  Many of the commands might be  used while you are speaking with a Tech Support Engineer. Commands follow

Cabrillo# show memory

Cabrillo# show stacks

Cabrillo# show buffers

Cabrillo# show arp

Cabrillo# show processes

Cabrillo# show processes cpu

Cabrillo# show tech-support

Step 8:  Finishing up

Once everything is working you may wish to add some commands to make your work easier.  Commands follow

Cabrillo(config)#  ip  host  Monterey  10.16.1.2 {Mapping names and IP addresses.)

Cabrillo(config)#  ip  name-server  ip-address    {Adding a name server.}

Cabrillo(config)#  no  ip domain-lookup  {When there is no domain server.}

Cabrillo(config)#  banner  motd  #  This is the Cabrillo  Router  #

{ note that the # symbol delineates the banner message}

Cabrillo(config-router)#  passive-interface  s  0    {When you do no want to advertise routing tables out of a specific interface.)

And don’t forget to...

Cabrillo#  show running-config

Cabrillo#  copy  running-config  startup-config

Miscellaneous  commands follow

Cabrillo#  ?  {This command can be used by itself or following at the end of any partial command line.}

Cabrillo>  enable

Cabrillo#  disable

Cabrillo#  configure terminal

Cabrillo(config)#  exit

Cabrillo(config-if)#  control-z

Cabrillo#  clock set hh:mm:ss day month year

Editing Commands follow

Control-A:  Moves to the beginning of the command line.

Control-E:  Moves to the end of the command line.

Esc-B:  Moves back one word.

Control F:  Moves forward one character.

Control-B:  Move back one character.

Esc F:  Moves forward one word.

Command History Commands follow

Control P or up arrow key:  Recalls last (previous command.

Control N or down arrow key:  Recalls most recent command

Tab key:  completes the entry.

Cabrillo#  show history

Cabrillo#  terminal history

Cabrillo#  terminal editing

Cabrillo#  no terminal editing