# Module 4 – Configuring a Router

# Module overview

Four Diagrams

Diagram 1, Tabular

Upon Completion of this module, the student will be able to perform tasks relating to the following:

- Discovering and Connecting to Neighbours

- Getting Information about Remote Devices

Diagram 2, Tabular

CCNA 640-801 Exam

Troubleshooting

•  Perform simple LAN troubleshooting

•  Troubleshoot IP addressing and host configuration

•  Troubleshoot a device as part of a working network

Diagram 3, Tabular

ICND 640-811 Exam

Troubleshooting

•  Perform LAN and VLAN troubleshooting

•  Troubleshoot IP addressing and host configuration

•  Troubleshoot a device as part of a working network

Diagram 4, Tabular

INTRO 640-821 Exam

Design and Support

•  Use a subset of Cisco IOS commands to analyze and report network problems

•  Use embedded Layer 3 through Layer 7 protocols to establish, test, suspend, or disconnect

connectivity to remote devices from the router console

Implementation and Operation

•  Establish communication between a terminal device and the router IOS, and

use IOS for system analysis.

•  Use commands incorporated within IOS to analyze and report network problems.

•  Use embedded data-link layer functionality to perform network neighbor discovery and analysis

from the router.

•  Use embedded Layer 3 through Layer 7 protocols to establish, test, suspend, or disconnect

connectivity to remote devices from the router console.

# Module 4.1: Discovering and Connecting to Neighbours

# Section 4.1.1: Introduction to CDP

Single Diagram

Diagram 1, Tabular

CDP

Description - Lists the Upper Layer Entry Addresses (TCP/IP, Novell IPX, AppleTalk, Others) then shows the 'Cisco Proprietary Data-Link Protocol' (CDP) , and thirdly Media Support  'SNAP' (LANs, Frame Relay, ATM, Others). The CDP sits between the upper layer entry addresses and the media support SNAP.

Text - CDP discovers and shows information about directly connected Cisco Devices

# Section 4.1.2: Information Obtained with CDP

Two Diagrams

Diagram 1, Relational

Description - Displays an operator on a router console. The router is connected to an IP/IPX Token Ring network and to two other routers (One router uses Router IP AppleTalk, the second Router IP CLNS).

Text - Single command summarises protocols and addresses on target (for example, neighbouring Cisco router)

Diagram 2, Command/Response

The 'show cdp neighbors' command displays the following information about the neighbouring devices - DeviceID, Local, Holdtime, Capability, Platform, and Port ID. In the situation in Diagram 1, there are two directly connected Cisco routers. - The following Txt is displayed.

"DeviceID 	Local 	Interfce  Holdtme Capablty Platform Port ID

Rt3		Ser0/1    	   152	       R		2500       Ser1

Rt1		Ser0/0              121         R            2620       Ser0/0  "

# Section 4.1.3: Implementation, Monitoring and Maintenance of CDP

Six Diagrams

Diagram 1, Tabular

CDP Command

Description - Matches the Command, Mode, and Purpose

Command: 'cdp run'

Mode: Glogal configuration mode

Purpose: Enables CDP globally on the router

Command: 'cdp enable'

Mode: Interface configuration mode

Purpose: Enables CDP on an Interface

Command: 'clear cdp counters'

Mode: Privileged EXEC mode

Purpose: Resets the Traffic Counters to Zero

Command: 'show cdp '

Mode: User or Privileged EXEC modes

Purpose: Displays the interval between transmissions of CDP advertisements, the number of seconds the CDP advertisement is valid for a given port, and the version of the advertisement

Command: 'show cdp entry {*  | device-name [*] [protocol | version]} '

Mode: User or Privileged EXEC modes

Purpose: Displays information about a specific neighbour. Display can be limited to protocol or version information.

Command: 'show cdp interface [type number]'

Mode: User or Privileged EXEC modes

Purpose: Displays information about interfaces on which CDP is enabled.

Command: 'show cdp'

Mode: Privileged EXEC mode

Purpose: Displays the type of device that has been discovered, the name of the device, the number and type of the local interface (Port), the number of seconds the CDP advertisement is valid for the port, the device type, the device product number, and the port ID. Issuing the 'detail ' keyword displays information on the native VLAN ID, the Duplex mode, and the VTP domain name associated with neighbour devices.

Diagram 2, Command/Response

The 'clear cdp counters' Command

The following text is displayed

"Rtl#show cdp traffic

CDP counters:

Total packets output: 6, Input: 6

Hdrsyntax: 0, Chksum error: 0, Encaps failed: 0

No memory: 0, Invalid packet: 0, Fragmented: 0

CDP version1 advertisements output: 0, Input: 0

CDP version2 advertisements output: 6, Input: 6

Rt1#clear cdp packets

Rt1#show traffic counters

CDP counters:

Total packets output: 6, Input: 6

Hdrsyntax: 0, Chksum error: 0, Encaps failed: 0

No memory: 0, Invalid packet: 0, Fragmented: 0

CDP version1 advertisements output: 0, Input: 0

CDP version2 advertisements output: 0, Input: 0

Rt1#   "

Diagram 3, Command/Response

The 'show cdp' Command

The following text is displayed

" CDP Version 1

Rt3#show cdp

Global CDP information:

Sending CDP packets every 60 seconds

Sending a holdtime value of 180 seconds

Rt3#

CDP Version 2

Rt1#show cdp

Global CDP information:

Sending CDP packets every 60 seconds

Sending a holdtime value of 180 seconds

Sending CDPv2 advertisements is enabled

Rt1#   "

Diagram 4, Command/Response

The 'show cdp entry' Command

The following text is displayed

" Rt1#show cdp entry Rt2

----------------------------------

Device ID: Rt2

Entry address(es):

IP address: 192.168.2.2

Platform: cisco 2621, Capabilities: Routrt

Interface: Serial0/0, PortID(outgoing pot) : Serial0/0

Holdtime: 139 sec

Version:

Cisco Internetwork Operating System Software

IOS ™ C2600 Software (C2600-DO3S-M), Version 12.0 (5) TI,

RELEASE

SOFTWARE(fcl)

Copyright(c) 1986-1999 br cisco System, Inc.

Compiled Tue 17-Aug-99 13:18 bycmong

Advertisement Version: 2

Rt1#   "

Diagram 5, Command/Response

The 'show cdp interface' Command

The following text is displayed

" Rt1# show cdp interface serial0/0

Serial0/0 is up, line protocol is up

Encapsulation HDLC

Sending CDP packets every 60 seconds

Holdtime is 180 seconds

Rt1#show cdp interface fastethernet0/0

Fastethernet0/0 is up, line protocol is up

Encapsulation ARPA

Sending CDP packets every 60 seconds

Holdtime is 180 seconds

Rt1#   "

Diagram 6, Command/Response

The 'show cdp neighbors' Command

The following text is displayed

"Rt2#show cdp neighbours

Compatability Codes: R-Router, T-TransBridge, B-Source

Route bridge, S-Switch, H-Host, I-IGMP, r-Repeater

DeviceID 	Local 	Interfce  Holdtme Capablty Platform Port ID

Rt3		Ser0/1    	   152	       R		2500       Ser1

Rt1		Ser0/0              121         R            2620       Ser0/0

Rt2#     "

# Section 4.1.4: Cteating a network map of the environment

Single Diagram

Diagram 1, Command/Response

Description -  Refer to Section 4.1.3 Diagram 6 (Repeated Diagram).

# Section 4.1.5: Disabling CDP

Single Diagram

Diagram 1, Command/Response

Disabling CDP

Description - Thefollowing text is displayed:-

" Rt1#show cdp

Global CDP information

Sending CDP packets every 60 seconds

Sending a hodtime value of 180 seconds

Sending CDPv2 advertisements is enables

Rt1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z

Rt1(config)#no cdp run

Rt1(config)#^Z

Rt1#show cdp

%CDP is not enabled

Rt1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z

Rt1(config)#cdp run

Rt1(config)#^Z     "

# Section 4.1.6: Troubleshooting CDP

Two Diagrams

Diagram 1, Tabular

Description - Matches Commands with their Descriptions

Command: 'clear cdp table'

Description: Deletes the CDP table of information about neighbours

Command: 'clear cdp counters'

Description: Resets the traffic counters to zero

Command: 'show cdp traffic'

Description: Displays CDP couters, including the number of packets sent and received and checksum errors.

Command: 'show debugging'

Description: Determines which types of debugging are enables

Command: 'debug cdp adjacency'

Description: CDP neighbour information

Command: 'debug cdp events'

Description: CDP events

Command: 'debug cdp ip'

Description: CDP IP information

Command: 'debug cdp packets'

Description: CDP packet-related information

Command: 'cdp timer'

Description: Specifies how often the Ciso IOS software sends CDP updates

Command: 'cdp holdtime'

Description: Specifies the hold time to be sent in the CDP update packet

Command: 'show cdp'

Description: Displays global CDP information, including timer and hold-time information.

Diagram 2, Command.Response

The 'show cdp traffic' Command

Description - The following text is displayed

" Rt2#show cdp traffic

CDP counters:

Total packets output: 526, Input: 323

Hdrsyntax: 0, Chksum error: 0, Encaps failed: 0

No memory: 0, Invalid packet: 0, Fragmented: 0

CDP version1 advertisements output: 168, Input: 153

CDP version2 advertisements output: 358, Input: 170  "

# Module 4.2: Discovering and Connecting to Neighbours

# Section 4.2.1: Telnet

Single Diagram

Diagram 1, Relational

Telnet Operates at the Application Layer

Description - Displays the OSI model and relates commands to layers

'telnet' - Application Layer

'ping' - Network Layer

'trace' -Network layer

'show ip route' - Network Layer

'show interface' - Physical, Data-link, and Network Layers

# section 4.2.2: Establishing and Verifying a Telnet connection

Two Diagrams

Diagram 1, Relational

Telnet Operations

Description - Displays a console connected to the Denver router. Two other routers are displayed - Paris and Tokyo. Only the Paris router has an IP address shown (131.108.100.152).

The commands to initiate and exit a telnet session are indicated

To initiate a session

"Denver>telnet Paris "

To Exit a session

"Paris>exit "

Diagram 2, Relational

Testing the Application Layer

Description - A console is connected to the York router, this router is connected throught the internet cloud to the Paris router. Telnet (Application layer) accomplishes this.

# Section 4.2.3: Disconnecting and Suspending Telnet Sessions

Single Diagram

Diagram 1, Command/Response - Relational

Telnet Operations

Description - Displays a console connected to the Denver router, two other routers are shown through a cloud, Paris and Tokyo, only the Paris router has a specified address (131.108.100.152)

The following 'How To' are indicated

Initiate a session

" Denver>telnet Paris "

End a Session

" Paris>exit "

Suspend a session

" Paris><Ctrl><Shift><6><x>

Denver>  "

Resume a session

" Denver><Enter> "

Disconnect a session

" Denver>disconnect Paris "

Display sessions

" Denver#show sessions

Conn		Host	Address		Idle	Conn Name

1		Paris	131.108.100.152 	0	Paris

2		Tokyo	126.102.57.63	0	Tokyo  "

# Section 4.2.4: Advanced telnet Operation

Two Diagrams

Diagram 1, Tabluar

Telnet Commands

Description - Matches Commands and their Purpose

Command 'Ctrl-Shift-6 then x'

Purpose: Suspends the current connection and returns to the EXEC prompt

Command ' Resume'

Purpose: Resumes the specific Telnet session that was previously created. The resume command requires a connection ID to specify which session to resume.

Diagram 2, Command/Response

Telnet Operations

" Denver>telnet Paris

Trying Paris (131.108.100.152)…Open

User Access Verification

Password:  xxxxx

Paris>      (User perssed Ctrl-Shift-6, then x)

Denver>telnet Tokyo

Trying Tokyo (127.102.57.63)…Open

User Access Verification

Password:  xxxxx

Tokyo>    (User perssed Ctrl-Shift-6, then x)

Denver>show sessions

Conn		Host	Address	Idle	Conn Name

1		131.108.100.152	0	Paris

2 		127.102.57.63	0	Tokyo   "

# Section 4.2.5: Alternative Connectivity Tests

Three Diagrams

Diagram 1, Relational/Command/Response

Testing with the 'ping' Command

Description - Displays two routers communication through a cloud using the 'Echo Request' and 'Echo Reply'. The following screen output is displayed

" Router>ping 172.16.1.5

Type escape sequence to abort.

Sending 5, 100 byte ICMP Echos to 172.16.1.5,

Timeout is 2 seconds:

!!!!!

Success rate is 100 percent,

Round-trip min/avg/max = 1/3/4 ms

Router>      "

Diagram 2, Relational

Testing with the 'ping' Command

Description - Displays two networks both with two workstations. The networks are interconnected by two routers (one for each network).

Audio - "To test the reachability of a device across an inter-network, a TCP/IP host can send an ICMP echo request. An ICMP echo request is generated by the 'ping' command."

Animation - A workstation on network A sends an 'echo request' the ICMP message travels through the routers to network B. The workstation in network B with the matching address respond with aa 'echo reply' (This travels back to the initiating workstation on network A).

Diagram 3, Relational/Command/Response

Testing with the 'traceroute' Command

Description - Displays four routers in an internet cloud, they are called 'York', 'London', 'Paris', and 'Rome'. The following IP addresses are indicated 'Paris: 172.16.16.2, Rome: 172.16.33.5, London: 172.16.12.3'. Arrows indicate the York-London-Paris-Rome route. York initiates a 'traceroute' command - the following response is indicated.

" York#traceroute ROME

Type escape to abort.

Tracing the route to ROME (172.16.33.5)

1 LONDON (172.16.12.3) 8 msec 8 msec 4 msec

2 PARIS (172.16.16.2) 8 msec 8 msec 8 msec

3 ROME (172.16.33.5) 8 msec 8 msec 4 msec

York#    "

# Section 4.2.6: Troubleshooting IP addressing Issues

Single Diagram

Diagram 1, Relational

Description - Displays the OSI model,

'telnet' is shown to operate at the 'Application' layer

'ping' is shown to operate at the 'Network' Layer

'traceroute' is shown to operate at the 'Network' Layer

# Summary

Single Diagram

Diagram 1, Tabular

- Cisco Discovery Protocol (CDP) is a layer 2 protocol that connects lower physical media and

upper network layer protocols.

- CDP is used to obtain information about neighbouring devices

- Telnet can be used to perform a test to determine whether or not access can be obtained from a

remote router.

- The results of the ping command can help evaluate the path-to-host reliability, delays over the

path, and whether the host can be reached or is functioning.

- The traceroute command is the ideal tool for finding where data is being sent in a network.