# Module 7 – Distance Vector Routing Protocols

# Module overview

Four Diagrams

Diagram 1, Tabular

Routing and Routing Protocols

After completing this chapter, students will be able to perform tasks relating to the following:

- Distance Vector Routing

- RIP

- IGRP

Diagram 2, Tabular

CCNA 640-801 Exam

This module will cover the following objectives

Planning and Designing

•  Select an appropriate routing protocol based on user requirements

Implementation and Operation

•  Configure routing protocols given user requirements Technology

Troubleshooting

•  Troubleshoot routing protocols

Technology

•  Evaluate the characteristics of routing protocols

Diagram 3, Tabular

ICND 640-811 Exam

This module will cover the following objectives

Planning and Designing

•  Select an appropriate routing protocol based on user requirements

Implementation and Operation

•  Configure routing protocols given user requirements

•  Implement a LAN

Troubleshooting

•  Troubleshoot routing protocols

Technology

•  Evaluate the characteristics of routing protocols

Diagram 4, Tabular

INTRO 640-821 Exam

This module will cover the following objectives

Technology

•  Describe the concepts associated with routing, and the different methods and protocols used to achieve it

# Module 7.1 – Distance Vector Routing

# Section 7.1.1: Distance Vector Routing Updates

Two Diagrams

Diagram 1, Relational/Descriptive

Distance Vector Topology Changes

Description - Displays two routers (A and B). Router A receives a periodic update, processes the update, and forwards the updated routing table to Router B. Router B does likewise, processing the update and forwarding appropriately.

Diagram 2, Descriptive

Routing Metric Components

Description - Displays the Routing metric components, Listed below

- Internetwork Delay

- Bandwidth

- Reliability

- Load

- Hop Count

# Section 7.1.2: Distance Vector Routing Loop Issues

Single Diagrams

Diagram 1, Relational

Problem: Routing Loops

Description – Displays five routers, A through E. Routers A, B, C, and D are connected in a diamond formation. Router E is connected to the network via Router A. A network that Router E supports goes down. The 'Network Unreachable' update is sent  through router E to Router A , and through the router network. As this is being forwarded to Router B and D from Router A, the router table in Router C (which hasn't yet received the 'Network Unreachable' update) still contains a forwarding address for the down network, As Router C forwards this table the path to reach the down network propagates with an ever increasing distance through the router network.

Text: "Alternate Routes, slow convergence, inconsistent routing."

# Section 7.1.3: Defining A Maximum Count

Two Diagrams

Diagram 1, Relational

Problem: Counting to Infinity

Description – Repeated Diagram, Refer to Diagram 1, Section 7.1.2.

Text - " Routing Loops increment the distance vector"

Diagram 2, Relational

Solution: Defining a Maximum

Description – Displays a similar diagram to the previous, however, by setting a maximum hop count the path is dropped (and not forwarded once a maximum Hop Count/Distance is reached ). This halts the endless looping.

# Section 7.1.4: Elimination Routing Loops Through Split-Horizon

Two Diagrams

Diagram 1, Relational

Solution: Split-Horizon

Description – Displays a similar diagram to Diagram 1, Section 7.1.2. This varies in that after Router A has forwarded the 'Network Unreachable' update the adjoining routers will not send to Router A any updates regarding the down Network, this again stops the routing loop.

Diagram 2, Relational

Simple Split-Horizon

Description – Displays two networks (A and B), connected via two Routers (Router 1 and Router 2 respectively).

Text  -  " With simple split-horizon, router table updates sent to a particular neighbour router should contain information about routes that were  learned from that neighbour. For example, suppose router 1 advertises that it has a route to network A. Router 2 receives the update from router 1 and inserts the information about Network A into its routing table. When router 2 sends a regular routing update, it does not include the entry for Network A in the update sent to router 1 because that route was learned from router 1 in the first place."

Animation - Displays Router 1 (Directly supporting Network A) forwarding routing table information about Network A to Router 2. Similarly Router 2 sends Router 1 routing table information about how to reach Network B (Which Router 2 Directly supports) This routing update that Router 2 sends to Router 1, does not include the routing information originally received from Router 1.

# Section 7.1.5: Route Poisoning

Single Diagram

Diagram 1, Relational

Route Poisoning

Description – Displays five Routers (A through E). Routers A, B, C, and E are connected in series. Router D has two connections (to both Router A, and Router B). The Network directly supported by Router E goes down. Router E initiate Route poisoning by entering a table entry metric of 16, which is unreachable.

# Section 7.1.6: Avoiding Routing Loops With Triggered Updates

Single Diagram

Diagram 1, Relational

Avoiding Routing Loops With Triggered Updates.

Description – Displays three routers connected in series (A, B and C). As a connected network fails, the change to the network is forwarded through all routers as soon as a change is noticed.

# Section 7.1.7Preventing Routing Loops with Hold-down Timers

Single Diagram

Diagram 1, Relational

Solution: Holddown Timers

Description – Displays a similar Diagram to Diagram 1, Section 7.1.2. In this scenario however the network supported by Router E is periodically up and down. Using holddown timers helps control the propagation of the changes through the network.

# Module 7.2 – RIP

# Section 7.2.1: RIP Routing Process

Single Diagram

Diagram 1, Listing

RIP Routing Process

Description – The key characteristics of RIP include the following

- It is a distance vector routing protocol

- Hop count is used as the metric for path selection

- If the hop count is greater than 15, the packet will be discarded

- By default, routing updates are broadcast every 30 seconds.

# Module 7.2.2 – Configuring RIP

Single Diagram

Diagram 1, Relational/Command Entry

Configuring RIP

Description - Displays three Routers (BHM, GAD, and BOAZ). These three routers are connected in series. The FastEthernet ports on Routers BHM, and BOAZ are connected to the Ethernet. The following Ports Addresses and command entries are displayed.

Router BHM

- FastEthernet Port 0/0 (To Ethernet) Address 10.0.0.254

- Serial Port S0/0 (To router GAD) Address 192.168.13.1

Router GAD

- Serial Port 0/1 (To Router BHM) Address 192.168.13.2

- Serial Port 0/0 (To Router BOAZ) Address 192.168.14.1

Router BOAZ

- Serial Port0/0 (To router GAD) Address 192.168.14.2

- FastEthernet Port 0/0 (To Ethernet) Address 172.31.31.1

Configuring Router BHM

"BHM(config)#router rip

BHM(config-router)#network 10.0.0.0

BHM(config-router)#network 192.168.13.0	"

Configuring Router GAD

"GAD(config)#router rip

GAD(config-router)#network 192.168.14.0

GAD(config-router)#network 192.168.13.0	"

Configuring Router BOAZ

"BOAZ(config)#router rip

BOAZ(config-router)#network 192.168.14.0

BOAZ(config-router)#network 172.31.0.0	"

# Module 7.2.3 – Using The ‘ip classless’ Command

Three Diagrams

Diagram 1, Relational

Using the ‘ip classless’ Command

Description – Displays a router configured without using the 'ip classless' command. If a message is to be forwarded for which the router has no forwarding information, then the message is dropped.

The following table is also displayed (Matching the outbound interface with the destination network).

Destination Network: 10.1.1.0

Outbound Interface: Fa 0/0

Destination Network: 10.3.3.0

Outbound Interface: Fa 0/1

Destination Network: 0.0.0.0

Outbound Interface: S 0/0

Diagram 2, Relational

Routing with ‘ip classless’

Description – Displays a router configured using the 'ip classless' command. If a message is to be forwarded for which the router has no forwarding information, then the message is now sent out to the best supernet route (In this case out Serial 0/0 to another router).

The Table in Diagram 1, Section 7.2.3 is also repeated

Diagram 3, Command Entries

Configure the ‘ip classless’ Command

Description – Displays the following text screenshot

"GAD#configure terminal

GAD(config)#ip classless	"

# Module 7.2.4: – Common RIP Configuration Issues

Six Diagrams

Diagram 1, Relational

Setting Holddown Timers

Description – Displays four routers in a diamond formation, The Holddown Timers on each router is set to 30 sec. Since the distance is four to complete the loop the Holddown timer should be set to a minimum value of 120 seconds (30 seconds for each router). Note that there is a conflict between convergence speed and the Holddown time.

Diagram 2, Relational

The ‘passive interface’ Command

Description - The following Text screenshot is diaplayed

" RouterB#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

R	192.168.1.0/24 [120/1] via 192.168.2.1, 00:00:02, Serial0/0

C	192.168.2.0/24 is directly connected, Serial0/0

C	192.168.3.0/24 is directly connected, FastEthernet0/0

RouterA#configure terminal

Enter configuration commands, one per line. End with Ctrl z.

RouterA#router rip

RouterA(config-router)#passive-interface s0/0

RouterA(config-router)#^Z

RouterB#clear ip route *

RouterB#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.2.0/24 is directly connected, Serial 0/0

C	192.168.3.0/24 is directly connected, FastEthernet0/0	"

Diagram 3, Tabular

The ‘passive interface’ Command

Description - Matches command with purpose

Command: "GAD(config-router)#passive-interface Fa0/0"

Purpose: Configures an interface to keep it from sending PRIP packets

Diagram 4, Tablular

The ‘neighbor’ Command

Description - Matches command with purpose

Command: "GAD(config-router)#neighbor ip address"

Purpose: Defines a neighbouring router with which to exchange routing information

Diagram 5, Tabular

Configuring the Router To Send and Receive Packets

Description - Matches commands with their purpose.

Command: "GAD(config-router)#version (1|2)"

Purpose: Configures the software to receive and send RIP Version 1 or Version 2 packets.

Command: "GAD(config-router)#ip rip send version 1"

Purpose: Configures an interface to accept only RIP Version 1 packets.

Command: "GAD(config-router)#ip rip send version 2"

Purpose: Configures an interface to send only RIP Version 2 packets.

Command: "GAD(config-router)#ip rip send version 1 2"

Purpose: Configures an interface to send only RIP Version 1 or 2 packets.

Diagram 6, Tabular

Controlling How Packets Received From An Interface

Description - Matches commands with their purpose.

Command: "GAD(config-if)#ip rip receive version 1"

Purpose: Configures an interface to accept only RIP version 1 packets.

Command: "GAD(config-router)#ip rip receive version 2"

Purpose: Configures an interface to accept only RIP Version 2 packets.

Command: "GAD(config-router)#ip rip receive version 1 2"

Purpose: Configures an interface to accept either RIP Version 1 or 2 packets.

# Section 7.2.5: Verifying RIP Configuration

Two Diagrams

Diagram 1, Command/Response

The ‘show ip protocols’ Command

Description - Displays the following Console Text.

"GAD#show ip protocols

Routing protocol is "rip"

Sending updates every 30 seconds, next due in 5 seconds

Invalid after 180 seconds, hold down 180, flushed after 240

Outgoing update filter list for all interfaces is

Incoming update filter list for all interfaces is

Redistributing Rip

Default version control: send version 1, receive any version

Interface		Send	 	Recv		Triggered RIP	Key-chain

FastEthernet0/0	1		1    2

Serial0/0		1		1    2

Routing for Networks:

192.168.1.0

192.168.2.0

Routing Information Sources:

Gateway	Distance	Last update

192.168.2.2	120		00:00:11

Distance: (default is 120)		"

The text in Italics are highlighted.

Diagram 2, Command/Response

The ‘show ip route’ Command

Description – Displays the following Console Text

"GAD#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.1.0/24 is directly connected, FastEthernet0/0

C	192.168.2.0/24 is directly connected, Serial0/0

R	192.168.3.0/24 {120/1} via 192.168.2.2, 00:00:07, Serial0/0   	"

Text in Italics are highlighted.

# Section 7.2.6: Troubleshooting RIP Update Issues

Three Diagrams

Diagram 1, Relational/Console Text

The 'debug ip rip' Command

Description – Repeated Diagram, refer to Diagram 1, Section 7.2.2. The following Console Text is also given.

"

BHM#debug ip rip

RIP event debugging is on

BHM#

00:45:33: RIP: received v1 update from 192.168.13.2 on Serial0/0

00:45:33:	192.168.14.0 in 1 hop

00:45:33:	172.31.0.0 in 2 hops

00:45:33:	172.29.0.0 in 15 hops

00:45:36:RIP: sending v1 update to 255.255.255.255 via Serial 0/0 (192.168.13.1)

00:45:36:	network 10.0.0.0. metric 1

00:45:36:RIP: sending v1 update to 255.255.255.255 via FastEthernet0/0 (10.0.0.254)

00:45:36:	network 192.168.13.0. metric 1

00:45:36:	network 192.168.14.0. metric 2

00:45:36:	network 172.31.0.0. metric 3

00:45:36:	network 172.29.0.0. metric 16	"

Diagram 2, Relational/Console Text

Discontiguous Subnetworks

Description - Repeated Diagram, refer to Diagram 1, Section 7.2.2. The following Console Text is also given.

"

BHM#debug ip rip

RIP event debugging is on

BHM#

7w2d: RIP: received v1 update from 192.168.13.2 on Serial0/0

7w2d:		192.168.14.0 in 1 hop

7w2d:		172.31.0.0 in 2 hops

7w2d: RIP: sending v1 update to 255.255.255.255 via Serial 0/0 (192.168.13.1)

7w2d:		network 172.31.0.0, metric 1

7w2d: RIP: sending v1 update to 255.255.255.255 via FastEthernet0/0 (10.0.0.254)

7w2d:		network 192.168.13.0. metric 1

7w2d		network 192.168.14.0. metric 2	"

Diagram 3, Relational/Console Text

Duplicate Subnetworks

Description – Repeated Diagram, refer to Diagram 1, Section 7.2.2. The following Console Text is also given.

"

BHM#debug ip rip

RIP event debugging is on

BHM#

7w2d: RIP: received v1 update from 192.168.13.2 on Serial0/0

7w2d:		192.168.14.0 in 1 hop

7w2d:		172.31.0.0 in 2 hops

7w2d: RIP: sending v1 update to 255.255.255.255 via Serial 0/0 (192.168.13.1)

7w2d:		network 172.31.0.0, metric 1

7w2d: RIP: sending v1 update to 255.255.255.255 via FastEthernet0/0 (10.0.0.254)

7w2d:		network 192.168.13.0. metric 1

7w2d		network 192.168.14.0. metric 2	"

# Section 7.2.7: Preventing Routing Updates Through An Interface

Single Diagram

Diagram 1, Relational

The 'passive-interface' Command

Description –  Displays five routers (A through E), Routers A, B, C, and D are connected in a diamond formation, Router E's only connection to the others is to Router A (Via Router E's Fa0/0 port). Router E does not want dynamic routes advertised so the following textual command is issued.

Text - "RouterE(config-router)#passive-interface Fa0/0"

# Section 7.2.8: Load Balancing with RIP

Two Diagrams

Diagram 1, Relational

RIP Load Balancing

Description – Two routers are trying to communicate, There are four possible routes, each with different speeds and distances. Illustrates the load balancing requirement,  and the need to use the available paths efficiently.

Diagram 2, Console Text

The ‘show ip route’ Command Output

Description – Displays a console screen containing the following text.

“Router#show ip route 10.0.0.0

Routing entry for 10.0.0.0/8

Known via "rip", distance 120, metric 1

Redistributing via rip

Advertised by rip (self originated)

Last update from 192.168.75.7 on Serial1, 00:00:00 ago

Routing Descriptor Blocks:

*	192.168.57.7, from 192.168.57.7, 00:00:18 ago, via Serial0

Route metric is 1, traffic share count is 1 192.168.75.7,

from 192.168.75.7, 00:000:00 ago, via Serial1

Route metric is 1, traffic share count is 1		"

# Section 7.2.9: Load Balancing Across Multiple Paths

Two Diagrams

Diagram 1, Tabular

Cisco IOS Administrative Distance

Description - Matches Administrative Distance Route source with the default distances

Administrative Distance Route Source: Connected Interface

Default Distance: 0

Administrative Distance Route Source: Static Route

Default Distance: 1

Administrative Distance Route Source: EIGRP summary mode

Default Distance: 5

Administrative Distance Route Source: External BGP

Default Distance: 20

Administrative Distance Route Source: EIGRP internal route

Default Distance: 90

Administrative Distance Route Source: IGRP

Default Distance: 100

Administrative Distance Route Source: OSPF

Default Distance: 110

Administrative Distance Route Source: IS-IS

Default Distance: 115

Administrative Distance Route Source: RIP

Default Distance: 120

Administrative Distance Route Source: EIGRP external mode

Default Distance: 170

Administrative Distance Route Source: Internal BGP

Default Distance: 200

Administrative Distance Route Source: Unknown

Default Distance: 255

Diagram 2, Relational

Three Ways To Get To Network X

Description – Displays five routers (A through E). Router E is connected to routers B,C,and D with administrative distances of 20, 10, and 20 respectively.  Router A is also connected to routers B,C,and D with administrative distances of 10, 10, and 25 respectively. (Note Routers B,C, and D have no other connections). Which route to take? when?...

# Section 7.2.10: Integrating Static Routes With RIP

Three Diagrams

Diagram 1, Relational/Console Text

RIP With Floating Static

Description – Displays two routers (GAD and BHM), both with 2 connections to each other. The S0/0 port of the GAD router uses a 1.544M connection speed, The BRI0/1 port of the GAD router communicates at dialup speeds. The address of the high speed port on the BHM router is 192.168.13.0/24, whereas the address of the dialup connection is 192.168.14.0/24. The following console text is displayed.

"

GAD#configure terminal

GAD(config)#ip route 172.16.0.0 255.255.0.0 192.168.14.2 130

GAD#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.13.0/24 is directly connected, Serial0/0

C	192.169.14.0/24 is directly connected, BRI0/1

R	172.16.0.0/16 {120/1} via 192.168.13.2, 00:00:24, Serial0/0	"

Diagram 2,  Relational/Console Text

Floating Static With Failed RIP Route

Description - Previous Diagram repeated however the high speed line is indicated to have failed. The following Console text is displayed

"

GAD#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.13.0/24 is directly connected, Serial0/0

C	192.169.14.0/24 is directly connected, BRI0/1

R	172.16.0.0/16 {120/1} via 192.168.14.2	"

Diagram 3, Tabular

Configuring a Static Route

Description  - Matches Command and Purpose

Command: ip route destination mask {interface/nexthop}

Purpose: Establish a static route

# Module 7.3 – IGRP

# Section 7.3.1: IGRP Features

Single Diagram

Diagram 1, Relational

IGRP Features

Description – Displays two workstations trying to communicate over a four router network. The shortest hop connection (Between routers) is the slowest (only 56K), whereas the other three inter-router connections are T1. Composite metrics select the path, and as speed is the primary consideration the communication path taken is seen to be the longest hop, where all four routers are involved in forwarding the data/messages.

# Section 7.3.2: IGRP Metrics

Two Diagrams

Diagram 1, Console Text

The 'show ip protocols' Command

Description - The following text is displayed

"

Router>show ip protocols

Routing protocol is igrp 300

Sending updates every 90 seconds, next due in 55 seconds

Invalid after 270 seconds, hold down 280, flushed after 360

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Default networks flagged in outgoing updates

Default networks accepted from incoming updates

IGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0

IGRP maximum hopcount 100

IGRP maximum metric variance 1

Redistributing igrp 300

Routing for networks:

183.8.0.0

144.253.0.0

Routing Information  Sources

Gateway		Distance	Last

Update

144.253.100.1	100	0:00:52

183.8.128.12		100	0:00:43

183.8.64.130		100	0:01:02

Distance: (default is 100)

--More--		"

Diagram 2, Console Text

The 'show ip route' Command

Description -  The following text is displayed

"

Router A#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.1.0/24 is directly connected, FastEthernet0/0

C	192.168.2.0/24 is directly connected, Serial0/0

I	192.168.3.0/24 {100/80135} via 192.168.2.2, Serial 0/0	"

# Section 7.3.3: IGRP Routes

Single Diagram

Diagram 1, Relational

Interior, System, and Exterior Routes

Description -  Displays two connected Autonomous Systems. The routers directly connecting the autonomous systems are termed 'External'. These exterior devices connect to system and internal devices within their respective systems.

# Section 7.3.4: IGRP Stability Features

Single Diagram

Diagram 1, Console Text

IGRP Timers

Description – Displays the response of the 'show ip protocols' command. The following text is highlighted to indicate relevance.

"Sending updates every 90 second, next due in 51 seconds

Invalid after 270 seconds, hod down 280, flushed after 630"

Section 7.3.5: Configuring IGRP

Two Diagrams

Diagram 1, Console Text

Configuring IGRP

Description – The following Text is displayed

"

RouterA(config)#router igrp 101

RouterA(config-router)#network 192.168.1.0

RouterA(config-router)#no router igrp 101	"

Diagram 2, Console Text

Configuring IGRP

Description – The following console text is displayed

"

RouterA(config)#router igrp 101

RouterA(config-router)#network 192.168.1.0

RouterA(config-router)#network 192.168.2.0

RouterB(config)#router igrp 101

RouterA(config-router)#network 192.168.2.0

RouterA(config-router)#network 192.168.3.0	"

# Section 7.3.6: Migrating RIP to IGRP

Seven Diagrams

Diagram 1, Console Text

Converting RIP to IGRP

Description – The following text is displayed

"

RouterA#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.1.0/24 is directly connected, Loopback0

C	192.168.2.0/24 is directly connected, Serial0/0

I	192.168.3.0/24 [120/1] via 192.168.2.2, 00:01:09, Serial 0/0

"

Diagram 2, Console Text

Converting RIP to IGRP

Description – The following text is displayed

"

RouterB#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

R	192.168.1.0/24 [120/1] via 192.168.2.1, 00:00:28, Serial0/0

C	192.168.2.0/24 is directly connected, Serial0/0

C	192.168.3.0/24 is directly connected, FastEthernet0/0	"

Diagram 3, Console Text

Converting RIP to IGRP

Description – The following text is displayed

"

Entered on Router A

RouterA#confgure terminal

RouterA(config)#router igrp 101

RouterA(config-router)#network 192.168.1.0

RouterA(config-router)#network 192.168.2.0

Entered on Router B

RouterB#configure terminal

RouterB(config)#router igrp 101

RouterA(config-router)#network 192.168.2.0

RouterA(config-router)#network 192.168.3.0 "

Diagram 4, Console Text

Converting RIP to IGRP

Description – The following text is displayed

" Routing protocol is “rip”

Sending updates every 30 seconds, next due in 2 seconds

Invalid after 180 seconds, hold down 180, flushed after 240

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Redistributing:rip

Default version control: send version 2, receive version 2

Interface	Send 	Recv	Triggered	RIP	Key-Chain

FastEthernet0/0   2	      2

Serial0/0	       2        2

Routing for networks:

192.168.1.0

192.168.2.0

Routing Information Sources:

Gateway	Distance	Last Update

192.168.2.2   120		00:00:21

Distance: (default is 120)

Routing Protocol is “igrp 101”

Sending updates every 90 seconds, next due in 45 seconds

Invalid after 270 seconds, hold down 280, flushed after 630

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Default networks flagged in outgoing updates

Default networks accepted from incoming updates

IGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0

IGRP maximum hopcount 100

IGRP maximum metric variance 1

Redistributing: igrp 101

Routing for networks:

192.168.1.0

192.168.2.0

Routing Information Sources:

Gateway	Distance	Last Update

192.168.2.2	100		00:00:38

Distance: (default is 100)			“

Diagram 5, Console Text

Converting RIP to IGRP

Description – The following text is displayed

"

RouterB#show ip protocols

Routing protocol is “rip”

Sending updates every 30 seconds, next due in 24 seconds

Invalid after 180 seconds, hold down 180, flushed after 240

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Redistributing:rip

Default version control: send version 2, receive version 2

Interface	Send 	Recv	Triggered	RIP	Key-Chain

FastEthernet0/0   2	      2

Serial0/0	       2        2

Routing for networks:

192.168.2.0

192.168.3.0

Routing Information Sources:

Gateway	Distance	Last Update

192.168.2.1   120		00:00:06

Distance: (default is 120)

Routing Protocol is “igrp 101”

Sending updates every 90 seconds, next due in 60 seconds

Invalid after 270 seconds, hold down 280, flushed after 630

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Default networks flagged in outgoing updates

Default networks accepted from incoming updates

IGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0

IGRP maximum hopcount 100

IGRP maximum metric variance 1

Redistributing: igrp 101

Routing for networks:

192.168.2.0

192.168.3.0

Routing Information Sources:

Gateway	Distance	Last Update

192.168.2.1	100		00:01:17

Distance: (default is 100)			“

Diagram 6, Console Text

Converting RIP to IGRP

Description – The following text is displayed

"

RouterA#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.1.0/24 is directly connected, FastEthernet0/0

C	192.168.2.0/24 is directly connected, Serial0/0

I	192.168.3.0/24 [100/80135] via 192.168.2.2, 00:00:36, Serial0/0  “	"

Diagram 7, Console Text

Converting RIP to IGRP

Description – The following text is displayed

"

RouterB#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

I	192.168.1.0/24 [100/8486] via 192.168.2.1, 00:01:05, Serial0/0

C	192.168.2.0/24 is directly connected, Serial0/0

C	192.168.3.0/24 is directly connected FastEthernet0/0	“

Section 7.3.7: Verifying IGRP Configuration

Five Diagrams

Diagram 1, Console Text

Verifying IGRP Configuration

Description – The following text is displayed

"

RouterA#show interface fa0/0

FastEthernet0/0 is up, line protocol is up

Hardware is AmdFE, address is 0009.7c89.5620 (bia 0009.7c89.5620)

Internet address is 192.168.1.1/24

MTU 1500 bytes, BW 100000 Kbit, DLY 100 usec,

Reliability 255/255, txload 1/255, txload 1/255

Encapsulation ARPA, loopback not set

Keepalive set (10 sec)

Half-Duplex, 100Mb/s, 100BaseTX/FX

ARP type: ARPA, ARP Timeout 04:00:00

Last input never, output 00:00:08, output hang never

Last clearing of “show interface” counters never

Queueing strategy: fifo

Output queue 0/40, 0 drops; input queue 0/75, 0 drops

5 minute input rate 0 bits/sec, 0 packets/sec

5 minute output rate 0 bits/sec, 0 packets/sec

0 packets input, 0 bytes

Received 0 broadcasts, 0 runts, 0 giants, o throttles

0 input errors, 0 CRC, 0 frame, 0 overruns, 0 ignored

0 watchdog, 0 multicast

0 input packets with dribble condition detected

113 packets output, 7701 bytes, 0 underruns

0 output errors, 0 collisions, 2 interface resets

0 babbles, 0 late collision, 0 deferred

0 lost carrier, 0 no carrier

0 output buffer failures, 0 output buffers swapped out  “

Diagram 2, Console Text

The 'show ip protocols' Output

Description – The following text is displayed

" RouterA#show ip protocols

Routing protocol is “igrp 101”

Sending updates every 90 seconds, next due in 72 seconds

Invalid after 270 seconds, hold down 280, flushed after 630

Outgoing update filter list for all interfaces is not set

Incoming update filter list for all interfaces is not set

Default networks flagged in outgoing updates

Default networks accepted from incoming updates

IGRP metric weight K1=1, K2=0, K3=1, K4=0, K5=0

IGRP maximum hopcount 100

IGRP maximum metric variance 1

Redistributing: igrp 101

Routing for networks:

192.168.1.0

192.168.2.0

Routing Information Sources:

Gateway	Distance	Last Update

192.168.2.2	100		00:00:07

Distance: (default is 100)		“

Diagram 3, Console Text

Verify the Network Statements

Description – The following text is displayed

"RouterA#show running-config | begin igrp

router igrp 101

network 192.168.1.0

network 192.168.2.0

!

no ip classless

no ip http server

!

line con 0

transport input none

line aux 0

line vty 0 4

password cisco

login

!

!

no schedule allocate

end.	“

Diagram 4, Console Text

Verify IP Addressing and Routing Tables

Description – The following text is displayed

"RouterA#show running-config interface fa0/0

Building configuration…

Current Configuration:

!

Interface FastEthernet0/0

Ip address 192.168.1.1 255.255.255.0

no ip directed-broadcast

end		“

Diagram 5, Console Text

Verify Routing Tables

Description – The following text is displayed

" RouterA#show ip route

Codes: C-connected, S-static, I-IGRP, R-RIP, M-mobile, B-BGP

D-EIGRP, EX-EIGRP external, O-OSPF, IA_OSPF inter area

N1-OSPF NSSA external type 1, N2-OSPF NSSA external type 2, E-EGP

i-IS-IS, L1-IS-IS level-1, L2-IS-IS level-2, ia-IS-IS inter area

*-candidate default, U - Per-user static route, o-ODR

P-periodic downloaded static route

Gateway of last resort is not set

C	192.168.1.0/24 is directly connected, Loopback0

C	192.168.2.0/24 is directly connected, Serial0/0

I	192.168.3.0/24 [100/80135] via 192.168.2.2, 00:01:00, Serial0/0	“

# Section 7.3.8:  Troubleshooting IGRP

Three Diagrams

Diagram 1, Console Text

Troubleshooting IGRP

Description – The following text is displayed

"RouterA#debug ip igrp events

IGRP event debugging is on

00:21:38: IGRP: sending update to 255.255.255.255

Via FastEthernet0/0 (192.168.1.1)

00:21:38: IGRP: Update contains 0 interior, 2 system, and 0 exterior routes

00:21:38: IGRP: Total routes in update 2

00:21:38: IGRP: sending update to 255.255.255.255 via Serial0/0 (192.168.2.1)

00:21:38: IGRP: Update contains 0 interior, 1 system, and 0 exterior routes.

00:21:38: IGRP: Total routes in update:1	“

Diagram 2, Console Text

Troubleshooting IGRP

Description – The following text is displayed

"RouterA#debug ip igrp transactions

IGRP protocol debugging is on

00:22:17: IGRP: received update from 192.168.2.2 on Serial0/0

00:22:17: IGRP: network 192.168.3.0, metric 80135 (neighbour 110)

00:23:07: IGRP: sending update to 255.255.255.255

via FastEthernet0/0 (192.168.1.1)

00:23:07: 	network 192.168.2.0, metric-80125

00:23:07:	network 192.168.3.0, metric-80135

00:23:07: IGRP: sending update to 255.255.255.255 via Serial0/0 (192.168.2.1)

00:23:07:	network 192.168.1.0, metric-110

Diagram 3, Console Text

Troubleshooting IGRP

Description – The following text is displayed

"

RouterA(config)#no router igrp 102

RouterA(config)#router igrp 101

RouterA(config-router)#network 192.168.1.0

RouterA(config-router)#network 192.168.2.0

00:27:50: IGRP: broadcasting request on FastEthernet0/0

00:27:50: IGRP: sending update to 255.255.255.255 via FastEthernet0/0 (192.168.1.1)

00:27:51: IGRP: Update contains 0 interior, 0 system, and 0 exterior routes

00:27:51: IGRP: Total routes in update: 0 – suppressing null update^Z

RouterA#

00:27:53: %SYS-5-CONFIG_I: Configured from console by console

00:27:58: IGRP: received update from 192.168.2.2 on Serial 0/0

00:27:58:	network 192.168.3.0, metric 80135 (neighbour 110)

00:27:58: IGRP: Update contains 0 interior, 1 system, and 0 exterior routes.

00:27:58: IGRP: Total routes in update: 1

00:27:58: IGRP: Total routes in update: 1

00:27:58: IGRP: sending update to 255.255.255.255 via FastEthernet0/0 (192.168.1.1)

00:27:58:  	network 192.168.2.0, metric=80125

00:27:58: : 	network 192.168.3.0, metric=80135

00:27:58: IGRP: Update contains 0 interior, 2 system, and 0 exterior routes

00:27:58: IGRP: Total routes in update: 2

00:27:58: IGRP: sending update to 255.255.255.255 via Serial0/0 (192.168.2.1)

00:27:58: 	network 192.168.1.0, metric=110

00:27:58: IGRP: Update contains 0 interior, 1 system, and 0 exterior routes.

00:27:58: IGRP: Total routes in update: 1

00:27:58: IGRP: received update from 192.168.2.2 on Serial0/0

00:27:58: 	network 192.168.3.0, metric 80135 (neighbour 110)

00:27:58: IGRP Update contains 0 interior, 1 system, and 0 exterior routes

00:27:58: IGRP: Total routes in update: 1

00:28:01: IGRP: sending update to 255.255.255.255 via FastEthernet0/0 (192.168.1.1)

00:28:01:  	network 192.168.2.0, metric=80125

00:28:01:  	network 192.168.3.0, metric=80135

00:28:01: IGRP: Update contains 0 interior, 2 system, and 0 exterior routes.

00:28:01: IGRP: Total routes in update: 2

00:28:01: IGRP: sending update to 255.255.255.255 via Serial0/0 (192.168.2.1)

00:28:01	network 192.168.1.0, metric=110

00:28:01: IGRP Update contains 0 interior, 1 system, and 0 exterior routes.

00:28:01: IGRP: Total routes in update: 1		“

# Summary

Single Diagram

Diagram 1, Tabular

Summary

Description - Lists the following points

-  Routing table updates occur periodically, when the topology in a distance vector protocol network changes

- RIP is a distance vector routing protocol

- RIP has evolved over the years from a classful routing protocol, RIP Version 1 (RIP v1), to a classless routing protocol, RIP Version 2 (RIP v2).

- IGRP is a distance vector routing protocol developed by Cisco