Cisci Systems

CCNA3 Module 3 Challenge Lab (NetLab Friendly)

Assigned Address Space 10.0.0.0/17

LoO: 10.0.64.0/20 Lo1: 10.0.80.0/20 LO2: 10.0.96.0/20 Lo3: 10.0.112.0/20

.0-0.4/30

Simulated

Webserver

1,0.0.1/32

LoO: 10.0.32.D/21 Lo1: 10.0,40,0/21 LoZ: 10.0,48,0/21 Lo3: 10.0,56.0/21

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| ISP | LoO | 1.0.0.1 | 255.255.255.255 |
|  |  | 2.0.0.1 | 255.255.255.252 |
| HQ |  | 2.0.0.2 | 255.255.255.252 |
|  | LoO |  |  |
|  | Lo1 |  |  |
|  | Lo2 |  |  |
|  | Lo3 |  |  |
| REMOTE |  |  |  |
|  | LoO |  |  |
|  | Lo1 |  |  |
|  | Lo2 |  |  |
|  | Lo3 |  |  |

1 -4      CCNA 3: Switching Basics and Intermediate Routing v 3.x -Module  3 Challenge Lab        Copyright © 2006, Cisco Systems, Inc.

Objective

Configure and Verify EIGRP, Default Routing and EIGRP Summary Routes

Step 1: Complete Topology and IP Addressing Table

•	Label the topology with the interface names of your particular routers (e.g. SO, S1, SO/0, etc.)

•	Fill in the table with the IP addresses you have chosen to use on the interfaces. Instructor Initials

Step 2: Cable the Topology and Basic Configuration

•	Choose three routers and cable them according to the topology. You will not need any LAN interfaces or switches for this lab. (If using NetLab, choose a three router pod)

•	Configure the routers according to your Instructor's required basic configuration including interface IP addresses. For this lab, you do not need to use the description command on the loopback interfaces. However, you must describe the WAN links. Copy configuration to NVRAM.

Instructor Initials

Step 3: Configure EIGRP Routing

•	Configure both HQ and REMOTE to use EIGRP as the routing protocol. Advertise the simulated LAN subnets and the WAN link between HQ and REMOTE. DO NOT advertise the 2.0.0.0/30 network. Turn off automatic summarization.

•	Verify that routing tables on HQ and REMOTE look like the ones below. If not, troubleshoot.

HQ#sh ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external, * -candidate  default

Gateway of last resort is not set

2.0.0.0/30 is subnetted, 1 subnets C       2.0.0.0 is directly connected, SerialO

| 07, | Seriall |
| --- | --- |
| 07, | Seriall |
| 07, | Seriall |
| 07. | Seriall |

10.0.0.0/8 is variably subnetted, 9 subnets, 3 masks C       10.0.0.4/30 is directly connected, Seriall D       10.0.40.0/21 [90/2297856] via 10.0.0.6, 00:02 D       10.0.32.0/21 [90/2297856] via 10.0.0.6, 00:02 D       10.0.56.0/21 [90/2297856] via 10.0.0.6, 00:02 D       10.0.48.0/21 [90/2297856] via 10.0.0.6, 00:02 C       10.0.64.0/20 is directly connected, LoopbackO C       10.0.80.0/20 is directly connected, Loopbackl C       10.0.96.0/20 is directly connected, Loopback2 C       10.0.112.0/20 is directly connected, Loopback3

REMOTE#show ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external, * -candidate  default

Gateway of last resort is not set

10.0.0.0/8 is variably subnetted, 9 subnets, 3 masks

C	10.0.0.4/30 is directly connected, SerialO

C	10.0.40.0/21 is directly connected, Loopbackl

C	10.0.32.0/21 is directly connected, LoopbackO

C	10.0.56.0/21 is directly connected, Loopback3

C	10.0.48.0/21 is directly connected, Loopback2

D	10.0.64.0/20 [90/2297856] via 10.0.0.5, 00:00:03, SerialO

D	10.0.80.0/20 [90/2297856] via 10.0.0.5, 00:00:03, SerialO

D	10.0.96.0/20 [90/2297856] via 10.0.0.5, 00:00:04, SerialO

D	10.0.112.0/20 [90/2297856] via 10.0.0.5, 00:00:04, SerialO

Instructor Initials

2 -4      CCNA 3: Switching Basics and Intermediate Routing v 3.x -Module  3 Challenge Lab        Copyright © 2006, Cisco Systems, Inc.

Step 4: Configure Static and Default Routing

•	Configure ISP with a static route pointing to the Assigned Address Space.

•	Configure HQ with a default route pointing to ISP.

•	Configure HQ to advertise the default route to REMOTE by entering the following commands: HQ(config)#ip  default-network  2.0.0.0

HQ(config)trouter eigrp 100

HQ(config-router)ttnetwork 2.0.0.0

•	Verify that routing tables on ISP, HQ and REMOTE look like the ones below. If not, troubleshoot.

ISP#show ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external,

-candidate  default

1.0.0.0/32 is subnetted, 1 subnets C       1.0.0.1 is directly connected, LoopbackO

2.0.0.0/30 is subnetted, 1 subnets C       2.0.0.0 is directly connected, SerialO

10.0.0.0/17 is subnetted, 1 subnets S       10.0.0.0 [1/0] via 2.0.0.2

HQ#show ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external, * -candidate  default

Gateway of last resort is 2.0.0.1 to network 0.0.0.0

|  | 2 | 0 0 0/30 |
| --- | --- | --- |
|  |  | 2.0.0.0 |
|  | 1 | 0.0.0.0/8 |
| c |  | 10.0.0. |
| D |  | 10.0.40 |
| D |  | 10.0.32 |
| D |  | 10.0.56 |
| D |  | 10.0.48 |
| C |  | 10.0.64 |
| C |  | 10.0.80 |
|  |  | 10.0.96 |
| C |  | 10.0.11 |
| S^ | 0 | .0.0.0/0 |

is subnetted, 1 subnets is directly connected, SerialO is variably subnetted, 9 subnets, 3 masks 4/30 is directly connected, Seriall

.0/21 [90/2297856] via 10.0.0.6, 00:25:05, Seriall .0/21 [90/2297856] via 10.0.0.6, 00:25:05, Seriall .0/21 [90/2297856] via 10.0.0.6, 00:25:05, Seriall .0/21 [90/2297856] via 10.0.0.6, 00:25:05, Seriall .0/20 is directly connected, LoopbackO .0/20 is directly connected, Loopbackl .0/20 is directly connected, Loopback2 2.0/20 is directly connected, Loopback3 [1/0] via 2.0.0.1

REMOTE#show ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external,

-candidate  default

Gateway of last resort is 1C

to network 2.

2.0.0.0/30 2.0.0.0 .0.0/8 3.0.0. . 40

10.0. 1 0  10 10 10 10 10 10 10 10

.0.48 .0.64 .0.80 .0. 96 .0.11

is subnetted, 1 subnets

0.32.0/21

[90/2681856] via 10.0.0.5, 00:20:30, SerialO is variably subnetted, 9 subnets, 3 masks 4/30 is directly connected, SerialO .0/21 is directly connected, Loopbackl is directly connected, LoopbackO .0/21 is directly connected, Loopback3 .0/21 is directly connected, Loopback2 .0/20 [90/2297856] via 10.0.0.5, 00:25:51, SerialO .0/20 [90/2297856] via 10.0.0.5, 00:25:52, SerialO .0/20 [90/2297856] via 10.0.0.5, 00:25:52, SerialO 2.0/20 [90/2297856] via 10.0.0.5, 00:25:52, SerialO

Instructor Initials

3 -4      CCNA 3: Switching Basics and Intermediate Routing v 3.x -Module  3 Challenge Lab        Copyright © 2006, Cisco Systems, Inc.

Step 5: Configure EIGRP Summary Routes

•	To reduce the size of the routing tables on both HQ and REMOTE, configure each router to summarize the simulated LANs (the loopback subnets) into one advertisement.

•	Summary route on HQ:

•	Summary route on REMOTE:

Verify that routing tables on HQ and REMOTE look like the ones below. If not, troubleshoot.

HQ#sh ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external, * -candidate  default

Gateway of last resort is 2.0.0.1 to network 0.0.0.0

*	2.0.0.0/30 is subnetted, 1 subnets C*      2.0.0.0 is directly connected, SerialO

10.0.0.0/8 is variably subnetted, 7 subnets, 4 masks C       10.0.0.4/30 is directly connected, Seriall

D       10.0.32.0/19 [90/2297856] via 10.0.0.6, 00:00:05, Seriall D       10.0.64.0/18 is a summary, 00:00:11, NullO C       10.0.64.0/20 is directly connected, LoopbackO C       10.0.80.0/20 is directly connected, Loopbackl C       10.0.96.0/20 is directly connected, Loopback2 C       10.0.112.0/20 is directly connected, Loopback3

S*	0.0.0.0/0 [1/0] via 2.0.0.1

REMOTE#show ip route

Codes: <some codes omitted>

D -EIGRP,  EX -EIGRP  external, * -candidate  default

Gateway of last resort is 10.0.0.5 to network 2.0.0.0

2.0.0.0/30 is subnetted, 1 subnets

2.0.0.0 [90/2681856] via 10.0.0.5, 00:01:43, SerialO

10.0.0.0/8 is variably subnetted, 7 subnets, 4 masks 10.0.0.4/30 is directly connected, SerialO 10.0.40.0/21 is directly connected, Loopbackl 10.0.32.0/19 is a summary, 00:02:41, NullO 10.0.32.0/21 is directly connected, LoopbackO 10.0.56.0/21 is directly connected, Loopback3 10.0.48.0/21 is directly connected, Loopback2 10.0.64.0/18 [90/2297856] via 10.0.0.5, 00:01:44, SerialO

Instructor Initials

Step 6: Verification and Documentation

•	Capture the following verifications to a "verify.txt" file.

o    Ping output from REMOTE pinging the Simulated Web Server

o    Capture show ip route on all three routers: ISP, HQ and REMOTE

o     Capture show  ip  eigrp  neighbors, show  ip  eigrp  topology and show  ip eigrp traffic on HQ and REMOTE

•	Capture the running configurations on all three routers to separate text files. Use the hostname of the router to name each text file.

•	Clean up the "verify.txt", "HQ.txt", "REMOTE.txt" and "ISP.txt" files. Add appropriate notes to assist in your studies.

Instructor Initials

4 -4      CCNA 3: Switching Basics and Intermediate Routing v 3.x -Module  3 Challenge Lab        Copyright © 2006, Cisco Systems, Inc.