Lab 2.3.1 Configuring the OSPF Routing Process

Router 1

Router 2

-

AreaO

| Router Router Designation Name |  |  | Enable Secret Password | Enable, VTY, and Console Passwords |  |  |  |  | Routing Protocol |  |  | Network Statements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router 1 | Berlin | class |  |  | Cisco |  |  | OSPF |  |  | 192.168.1.128 192.168.15.0 |
| Router 2 | Rome | class |  |  | Cisco |  |  | OSPF |  |  | 192.168.15.0 192.168.0.0 |
| Router IP Host FastEthernet 0 Designation Table Address/ Entry Subnet Mask |  |  |  |  | Interface Type Serial 0 |  |  | Serial 0 Interface Address/Subnet Type Serial 1 Mask |  |  |  |  | Serial 1 ^ Add ress/S u b net Mask |
| Router 1 | Rome | 192.168.1.129/26 | DCE |  |  | 192.168.15.1/30 |  |  | NA | No address |
| Router 2 | Berlin | 192.168.0.1/24 | DTE |  |  | 192.168.15.2/30 |  |  | NA | No address |

Note: The IP Host Table Entry column contents indicate the name(s) of the other router(s) in the IP host table. Straight-through cable	¦^^^^^^^^^^^^^—	Console (Rollover)

Serial cable

Crossover cable

Objective

•	Setup an IP addressing scheme for OSPF area 0.

•	Configure and verify Open Shortest Path First (OSPF) routing.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in the lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

1 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.1

Copyright © 2003, Cisco Systems, Inc.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname as shown in the chart. Then configure the console, virtual terminal and enable passwords. Next configure the interfaces according to the chart. Finally, configure the IP hostnames. Do not configure the routing protocol until specifically told to. If there are any problems configuring the router basics, refer to prior lab "Review of Basic Router Configuring with RIP".

Step 2 Save the configuration information from the privileged EXEC command mode

BERLIN#copy running-config startup-config

Destination filename [startup-config]? [Enter]

a.   Why save the running configuration to the startup configuration?

Step 3 Configure the hosts with the proper IP address, subnet mask, and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 98, check by using Start >Run > winipcfg. If running Windows 2000, check by using the ipconf ig command in a DOS window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using OSPF as the routing protocol.

Step 4 View the routers configuration and interface information

a.	At the privileged EXEC mode prompt type:

Berlin#show running-config

b.	Using the show ip interface brief command, check the status of each interface.

c.	What is the state of the interfaces on each router? Berlin:

FastEthernet 0:

Serial 0:

Serial 1:

Rome:

FastEthernet 0:

Serial 0:

d.	Ping from one of the connected serial interfaces to the other.

Was the ping successful?

e.   If the ping was not successful, troubleshoot the router configuration, until the ping is successful.

2 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.1	Copyright © 2003, Cisco Systems, Inc.

Step 5 Configure OSPF routing on router Berlin

a.	Configure an OSPF routing process on router Berlin. Use OSPF process number 1 and ensure all networks are in area 0.

Berlin(config)#router ospf 1

Berlin(config-router)#network 192.168.1.128 0.0.0.63 area 0

Berlin(config-router)#network 192.168.15.0 0.0.0.3 area 0

Berlin(configrouter)#end

b.	Examine the routers running configurations files.

c.	Did the IOS version automatically add any lines under router OSPF 1 ?

d.	If so, what did it add?

e.	If there were no changes to the running configuration, type the following commands:

Berlin(config)#router ospf 1

Berlin(configrouter)#logadjacency-changes

Berlin(configrouter)#end

f.	Show the routing table for the Berlin router. Berlin#show  ip   route

g.	Are there any entries in the routing table?

h.   Why?

Step 6 Configure OSPF routing on router Rome

a.	Configure an OSPF routing process on router Rome. Use OSPF process number 1 and ensure all networks are in area 0.

Rome(config)#router ospf 1

Rome(config-router)#network 192.168.0.0 0.0.0.255 area 0

Rome(config-router)#network 192.168.15.0 0.0.0.3 area 0

Rome(configrouter)#end

b.	Examine the Rome running configuration files.

c.	Did the IOS version automatically add any lines under router OSPF 1 ?

d.	If so, what did it add?

e.	If there were no changes to the running configuration, type the following commands:

Rome(config)#router ospf 1

Rome(configrouter)#logadjacency-changes

Rome(configrouter)#end

f.	Show the routing table for the Rome router:

3 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.1	Copyright © 2003, Cisco Systems, Inc.

Rome#show ip route

g.   Are there any OSPF entries in the routing table now?

h.   What is the metric value of the OSPF route?

i.    What is the VIA address in the OSPF route?

j.    Are routes to all networks shown in the routing table?

k.   What does the O mean in the first column of the routing table?

Step 7 Test network connectivity

a.	Ping the Berlin host from the Rome host. Was it successful?

b.	If not troubleshoot as necessary.

Once the previous steps are completed, log off by typing exit, and turn the router off. Then remove and store the cables and adapter.

4 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.1	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted for a password, enter class. If that does not work, ask the instructor for assistance.

Router>enable At the privileged EXEC mode, enter the command erase startup-config.

Router#erase  startup-config The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase  of  nvram:   complete Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload The responding line prompt will be:

System  configuration  has   been  modified.   Save?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Proceed  with  reload?    [confirm] Press Enter to confirm. In the first line of the response will be:

Reload  requested by  console. After the router has reloaded the line prompt will be:

Would  you   like   to   enter   the   initial   configuration  dialog?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Press  RETURN  to  get   started! Press Enter. Now the router is ready for the assigned lab to be performed.

5 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.1	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

6 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.1

Copyright © 2003, Cisco Systems, Inc.

ClSCJ  StSTEHS

Lab 2.3.2 Configuring OSPF with Loopback Addresses

Router 1

Or

Router 3

| Router Designation | Router Name | Enable Secret Password | Enable, VTY, and Console Passwords |  |  | Routing Protocol | OSPF Network 7 Priority Statements |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router 1 | London | class | Cisco |  |  | OSPF | 1 | 192.168.1.0 |
| Router 2 | Ottawa | class | Cisco |  |  | OSPF | 1 | 192.168.1.0 |
| Router 3 | Brasilia | class | Cisco |  |  | OSPF | 1 | 192.168.1.0 |
| Router Designation |  |  | IP Host Table Entry | FastEthernet 0 Address/ Subnet Mask |  |  |  |  | Loopback Address/ Subnet Mask |  |  |
| Router 1 |  |  | Ottawa Brasilia |  |  | 192.168.1.1/24 |  |  | 192.168.31.11/32 |  |  |
| Router 2 |  |  | London Brasilia |  |  | 192.168.1.2/24 |  |  | 192.168.31.22/32 |  |  |
| Neuter 3 |  |  | London Ottawa |  |  | 192.168.1.3/24 |  |  | 192.168.31.33/32 |  |  |

Note: The IP Host Table Entry column contents indicate the narne(s) of the other router(s) in the IP host table.

Straight-through cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Configure routers with a Class C IP addressing scheme.

•	Observe the election process for designated routers (DR) and backup designated routers (BDR) on the multiaccess network.

•	Configure loopback addresses for Open Shortest Path First (OSPF) stability.

•	Assign each OSPF interface a priority to force the election of a specific router as DR.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in the lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

1 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2

Copyright © 2003, Cisco Systems, Inc.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname as shown in the chart. Then configure the console, virtual terminal and enable passwords. Next configure the interfaces according and the IP hostnames. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router Configuring with RIP". Do not configure loopback interfaces and routing protocol yet.

Step 2 Save the configuration information for all the routers

Why save the running configuration to the startup configuration?

Step 3 Configure the hosts with the proper IP address, subnet mask and default gateway

Each workstation should be able to ping all of the attached routers. That is because they are all part of the same subnetwork. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 98, check by using Start > Run > winipcf g. If running Windows 2000, check by using the ipconf ig command in a DOS window.

Step 4 View the routers configuration and interface information

a.	At the privileged EXEC mode prompt type: show running-config

b.	Using the show ip interface brief command, check the status of each interface.

c.	What is the state of the interfaces on each router? London:

FastEthernet 0:

Serial 0:

Serial 1:

Ottawa:

FastEthernet 0:

Serial 0:

Serial 1:

Brasilia:

FastEthernet 0:

Serial 0:

Serial 1:

Step 5 Verify connectivity of the routers

a.	Ping all of the connected FastEthernet interfaces from each other.

b.	Were the pings successful?

c.    If the pings were not successful, troubleshoot the router configuration, until the ping is successful.

2 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2	Copyright © 2003, Cisco Systems, Inc.

Step 6 Configure OSPF routing on router London

a.	Configure an OSPF routing process on the router London. Use OSPF process number 1 and ensure all networks are in area 0.

London(config)#router ospf 1

London(config-router)#network 192.168.1.0 0.0.0.255 area 0

London(configrouter)#end

b.	Examine the London router running configuration file.

c.	Did the IOS version automatically add any lines under router OSPF 1 ?

d.	If there were no changes to the running configuration, type the following commands.

London(config)#router ospf 1

London(configrouter)#logadjacency-changes

London(configrouter)#end

e.	Show the routing table for router: London#show  ip   route

f.	Are there any entries in the routing table?

g.	Why?

Step 7 Configure OSPF routing on router Ottawa

a.	Configure an OSPF routing process on the router Ottawa. Use OSPF process number 1 and ensure all networks are in area 0.

Ottawa(config)#router ospf 1

Ottawa(config-router)#network 192.168.1.0 0.0.0.255 area 0

Ottawa(configrouter)#end

b.	Examine the Ottawa running configuration file.

c.	Did the IOS version automatically add any lines under router OSPF 1 ?

d.   If there were no changes to the running configuration, type the following commands.

Ottawa(config)#router ospf 1

Ottawa(configrouter)#logadjacency-changes

Ottawa(configrouter)#end

Step 8 Configure OSPF routing on router Brasilia

a.   Configure an OSPF routing process on the router Brasilia. Use OSPF process number 1 and ensure all networks are in area 0.

Brasilia(config)#router ospf 1

Brasilia(config-router)#network 192.168.1.0 0.0.0.255 area 0

Brasilia(configrouter)#end

3 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2	Copyright © 2003, Cisco Systems, Inc.

b.	Examine the Brasilia router running configuration file.

Did the IOS version automatically add any lines under router OSPF 1 ?

c.	What did it add?

d.   If there were no changes to the running configuration, type the following commands:

Brasilia(config)#router ospf 1

Brasilia(configrouter)#logadjacency-changes

Brasilia(configrouter)#end

Step 9 Test network connectivity

a.	Ping the Brasilia router from the London router. Was it successful?

b.	If not troubleshoot as necessary.

Step 10 Show OSPF adjacencies

a.	Type the command show ip ospf neighbor on all routers to verify that the OSPF routing has formed adjacencies.

b.	Is there a designated router identified?

c.	Is there a backup designated router?

d.	Type the command show ip ospf neighbor detail for more information.

e.	What is the neighbor priority of 192.168.1.1 from router Brasilia?

f.	What interface is Identified as being part of Area 0?

Step 11 Configure the loopback interfaces

Configure the loopback interface on each router to allow for an interface that will not go down due to network change or failure. This task is performed by typing interface loopback # at the global configuration mode prompt, where the # represents the number of the loopback interface from 0 2,147,483,647.

London(config)#interface loopback 0

London(config-if)#ip address 192.168.31.11 255.255.255.255

London(configrouter)#end

Ottawa(config)#interface loopback 0

Ottawa(config-if)#ip address 192.168.31.22 255.255.255.255

Ottawa(configrouter)#end

Brasilia(config)#interface loopback 0

Brasilia(config-if)#ip address 192.168.31.33 255.255.255.255

Brasilia(configrouter)#end

Step 12 Save the configuration information for all the routers

After saving the configurations on all of the routers, power them down and back up again.

4 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2	Copyright © 2003, Cisco Systems, Inc.

Step 13 Show OSPF adjacencies

a.	Type the command show ip ospf neighbor on all routers to verify that the OSPF routing has formed adjacencies.

b.	Is there a designated router identified?

c.	Write down the router ID and link address of the DR.

d.	Is there a backup designated router?

e.	Write down the router ID and link address of the BDR.

f.	What is the third router referred to as?

g.   Write down that Routers ID and link address.

h.   Type the command show ip ospf neighbor detail for more information.

i.    What is the neighbor priority of 192.168.1.1 from router Brasilia?

j.    What interface is Identified as being part of Area 0?

Step 14 Verify OSPF interface configuration

a.	Type show ip ospf interface fastethernet 0 on the London router.

b.	What is the OSPF state of the interface?

c.	What is the default priority of the interface?

d.	What is the network type of the interface? _

Step 15 Configure London to always be the DR

To ensure that the London router always becomes the DR for this multi-access segment, the OSPF priority must be set. London is the most powerful router in the network and so best suited to become DR. To assign the London loopback a higher IP address is not advised, as the numbering system has advantages for troubleshooting. Also London is not to act as DR for all segments to which it may belong. Set the priority of the interface to 50 on the London router only.

London(config)#interface Fastethernet 0/0 London(config-if)#ip ospf priority 50

London(configif)#end

Display the priority for Interface fastethernet 0/0.

London#show ip ospf interface fastethernet 0/0

Step 16 Watch election process

a.	To watch the OSPF election process restart all of the routers using the reload command. Be sure to save the running config before restarting the routers. As soon as the router prompt is available type:

Ottawa>enable

Ottawa#debug ip ospf events

b.	Which router was elected DR?

5 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2	Copyright © 2003, Cisco Systems, Inc.

c.	Which router was elected BDR?

d.	Why?

e.   To turn off all debugging type undebug all.

Step 17 Show OSPF Adjacencies

a.	Type the command show ip ospf neighbor on the Ottawa router to verify that the OSPF routing has formed adjacencies.

b.	What is the priority of the DR?

Once the previous steps are completed log off by typing exit and turn the router off. Then remove and store the cables and adapter.

6 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted for a password, enter class. If that does not work, ask the instructor for assistance.

Router>enable At the privileged EXEC mode, enter the command erase startup-config.

Router#erase  startup-config The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase  of  nvram:   complete Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload The responding line prompt will be:

System  configuration  has   been  modified.   Save?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Proceed  with  reload?    [confirm] Press Enter to confirm. In the first line of the response will be:

Reload  requested by  console. After the router has reloaded the line prompt will be:

Would  you   like   to   enter   the   initial   configuration  dialog?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Press  RETURN  to  get   started! Press Enter. Now the router is ready for the assigned lab to be performed.

7 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

8 -8          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.2

Copyright © 2003, Cisco Systems, Inc.

Cisco Ststems

Lab 2.3.3 Modifying OSPF Cost Metric

iJr~^&~f»r^-IS""" JJL

Router 1

Router 2

Area 0

| Router Designation | Router Name | Enable Secret Password | Enable, VTY, and Console Passwords | Routing Protocol | Network Statements |
| --- | --- | --- | --- | --- | --- |
| Router 1 | Cairo | class | CISCO | OSPF | 192.168.1.0 |
| Router 2 | Moscow | class | Cisco | OSPF | 192.168-1-0 192.166.0.0 |

Router

IP Host FastEthernet 0 Interface Serial 0

Interface

Serial 1

Designation Table     Addressr	Type         Address/Subnet Type Serial 1    Address/Subnet

Entry     Subnet Mask     Serial 0     Mask	 Mask

No address Mo address

Router 1	I Moscow I 192.168-1 129/26  I DCE	I 192.168.1.1/30	NA

Router2	Cairo        192.168.0.1/24        DTE	192.168.1.2/30         I NA

Note: The IP Host Table Entry column contents indicate the name(s) of the other routers} in the IP host table.

Straight-through cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Setup an IP addressing scheme for Open Shortest Path First (OSPF) area.

•	Configure and verify OSPF routing.

•	Modify OSPF cost metric on an interface.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in the lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname, console, virtual terminal, and enable passwords. Next configure the interfaces and IP hostnames according to the

1 -7

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3

Copyright © 2003, Cisco Systems, Inc.

chart. Do not configure the routing protocol until specifically told to. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router Configuration with RIP"

Step 2 Save the configuration information from the privileged EXEC command mode

Cairo#copy running-config startup-config

Destination filename [startup-config]?[Enter]

Moscow#copy running-config startup-config

Destination filename [startup-config]?[Enter]

Why save the running configuration to the startup configuration?

Step 3 Configure the hosts with the proper IP address, subnet mask and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 9x/ME, check by using Start > Run > winipcfg. If running Windows NT/2000/XP, check by using the ipconf ig command in a Command Prompt window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using OSPF as the routing protocol.

Step 4 View the routers configuration and interface information

a.	At the privileged EXEC mode type:

Cairo#show running-config

b.	Using the show ip interface brief command, check the status of each interface.

c.	What is the state of the interfaces on each router? Cairo:

FastEthernet 0:

Serial 0:

Moscow:

FastEthernet 0: Serial 0:

d.	On a router, ping the serial interface of the other router.

e.	Was the ping successful?

f.    If the ping was not successful, troubleshoot the router configuration until the ping is successful.

Step 5 Configure OSPF routing on router Cairo

a.   Configure OSPF routing on each router. Use OSPF process number 1 and ensure all networks are in area 0.

Cairo(config)#router ospf 1

Cairo(config-router)#network 192.168.1.128 0.0.0.63 area 0

2 -7          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3	Copyright © 2003, Cisco Systems, Inc.

Cairo(config-router)#network 192.168.1.0 0.0.0.3 area 0

Cairo(configrouter)#end

b.	Examine the running configuration file.

c.	Did the IOS version automatically add any lines under router OSPF 1?

d.	What did it add?

e.	If there were no changes to the running configuration, type the following commands:

Cairo(config)#router ospf 1

Cairo(configrouter)#logadjacency-changes

Cairo(configrouter)#end

f.	Show the routing table for the Cairo router. Cairo#show  ip   route

g.	Are there any entries in the routing table?

h.   Why?

Step 6 Configure OSPF routing on router Moscow

a.	Configure OSPF routing on each router. Use OSPF process number 1 and ensure all networks are in area 0.

Moscow(config)#router ospf 1

Moscow(config-router)#network 192.168.0.0 0.0.0.255 area 0

Moscow(config-router)#network 192.168.1.0 0.0.0.3 area 0

Moscow(configrouter)#end

b.	Examine the running configuration file.

c.	Did the IOS version automatically add any lines under router OSPF 1?

d.   If there were no changes to the running configuration, type the following commands:

Moscow(config)#router ospf 1

Moscow(configrouter)#logadjacency-changes

Moscow(configrouter)#end

Step 7 Show the routing table entries

a.	Show the routing table entries for the Cairo router.

Cairo#show ip route

b.	Are there any OSPF entries in the routing table now?

c.	What is the metric value of the OSPF route?

d.	What is the VIA address in the OSPF route?

e.   Are routes to all networks shown in the routing table?

f.    What does the O mean in the first column of the routing table?

3 -7          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3	Copyright © 2003, Cisco Systems, Inc.

Step 8 Test network connectivity

a.	Ping the Cairo host from the Moscow host. Was it successful?

b.	If not troubleshoot as necessary.

Step 9 Look at the OSPF cost on the Cairo router interfaces

| Link Bandwidth | Default OSPF Cost |
| --- | --- |
| 56 Kbps | 1785 |
| T1 | 65 |
| 10Mbps | 10 |
| 16-Mbps Token-ring | 6 |
| FDDI/Fast Ethernet | 1 |

a.	Show the properties of the Cairo router serial and FastEthernet interfaces using the show interfaces command.

b.	What is the default bandwidth of the interfaces?

c.	Serial Interface:

d.   FastEthernet Interface:

e.	Calculate the cost of a link divide 10 by the bandwidth in bps.

f.	Serial Interface:

g.	FastEthernet Interface:

Step 10 Record the OSPF cost of the serial and FastEthernet interfaces

a.	Using the show ip ospf interface command, record the OSPF cost of the serial and Fast Ethernet interfaces.

b.	OSPF cost of Serial Interface:

c.   OSPF cost of Ethernet Interface:

d.   Do these agree with the calculations?

Step 11 Manually set the cost on the serial interface

On the Serial interface of the Cairo router, set the OSPF cost to 1562 by typing ip ospf cost 1562 at the serial interface configuration mode prompt.

Step 12. Verify cost

a.	Note that it is essential that all connected links agree about the cost for consistent calculation of the shortest path first algorithm (SPF) in an area.

b.	Verify that the interface OSPF cost was successfully modified.

c.	Reverse the effect of this command by entering in interface configuration mode the command no ip ospf cost.

d.	Verify that the default cost for the interface has returned.

4-7

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3

Copyright © 2003, Cisco Systems, Inc.

e.	Enter the command bandwidth 2 000 at the serial 0 interface configuration mode.

f.	Record the new OSPF cost of the Serial interface.

g.   Can the OSPF cost of an Ethernet interface be modified in this way?

h.   The speed can be set on an Ethernet interface. Will this affect the OSPF cost of that interface?

i.    Verify or explain the above answer.

j.     Reset the bandwidth on the serial interface using the no bandwidth 2 000 at the serial 0 interface configuration mode.

Once completion previous steps are completed, logoff by typing exit and turn the router off. Then remove and store the cables and adapters.

5 -7          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted for a password, enter class. If that does not work, ask the instructor for assistance.

Router>enable At the privileged EXEC mode, enter the command erase startup-config.

Router#erase  startup-config The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase  of  nvram:   complete Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload The responding line prompt will be:

System  configuration  has   been  modified.   Save?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Proceed  with  reload?    [confirm] Press Enter to confirm. In the first line of the response will be:

Reload  requested by  console. After the router has reloaded the line prompt will be:

Would  you   like   to   enter   the   initial   configuration  dialog?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Press  RETURN  to  get   started! Press Enter. Now the router is ready for the assigned lab to be performed.

6 -7          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

7-7

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.3

Copyright © 2003, Cisco Systems, Inc.

Cisco Systems

Lab 2.3.4 Configuring OSPF Authentication

Router 1

Router 2

fl

Area 0

| Router Designation | Router Name | Enable Secret Password | Enable, VTYf and Console Passwords | Routing Protocol | Network Statements |
| --- | --- | --- | --- | --- | --- |
| Router 1 | Dublin | class | CJSCO | OSPF | 192.168.1.0 |
| Router 2 | Washington | class | CISCO | OSPF | 192.168.1.0 192.168.0.0 |

Router	IP Host     FastEthernet 0 Interface Serial 0	Interface        Loopback 0

Designation Table        Address/	Type        Address/Subnet Type Serial 1 Address/Subnet

Entry        Subnet Mask     Serial 0    Mask	 Mask

| Router 1 | Washington | 192.166.1.129/26 | DCE | 192. IBS. 1.1/30 | NA | 19Z. 168.31.11/32 |
| --- | --- | --- | --- | --- | --- | --- |
| Router 2 | Dublin | 192168.0.1/24 | DIE | 192,188,1,2/30 | NA | 192.168.31.22/32 |

Note: The IP Hosl Table Entry column contents Indicate the name(s) of the other nouter(s) in the IP host table.

Straight-through cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Setup an IP addressing scheme for Open Shortest Path First (OSPF) area.

•	Configure and verify OSPF routing.

•	Introduce OSPF authentication into the area.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname, console, virtual terminal and enable passwords. Next configure the interfaces and IP hostnames according to the

1 -5

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.4

Copyright © 2003, Cisco Systems, Inc.

chart. Do not configure the routing protocol until specifically told to. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router Configuration with RIP"

Step 2 Save the configuration information from the privileged EXEC command mode

Dublin#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Washington#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Why save the running configuration to the startup configuration?

Step 3 Configure the hosts with the proper IP address, subnet mask and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 9x/ME, check by using Start > Run > winipcfg. If running Windows NT/2000/XP, check by using the ipconf ig command in a Command Prompt window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using OSPF as the routing protocol.

Step 4 Verify connectivity

a.	On a router, ping the serial interface of the other router.

b.	What the ping successful?

c.    If the ping was not successful, troubleshoot the routers configurations, until the ping is successful.

Step 5 Configure OSPF routing on both routers

a.	Configure OSPF routing on each router. Use OSPF process number 1 and ensure all networks are in area 0. Refer to the lab, "Configuring Loopback Interfaces" for review on configuring OSPF routing if necessary.

b.	Examine the Dublin router running configuration file. Did the IOS version automatically add any lines under router OSPF 1 ?

c.	Show the routing table for the Dublin router.

Dublin#show ip route

d.	Are there any entries in the routing table?

e.	Why?

Step 6 Test network connectivity

a.	Ping the Dublin host from the Washington host. Was it successful?

b.	If not troubleshoot as necessary.

2 -5          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.4	Copyright © 2003, Cisco Systems, Inc.

Step 7 Setup up OSPF authentication

a.	OSPF authentication is being established on the routers in the network. First, introduce authentication only on the Dublin router.

b.	In the interface configuration mode on Serial 0, enter the command ip ospf message-digest-key   1 md.5  7  asecret.

Dublin(config)#interface Serial 0

Dublin (config-if) #ip ospf message-digest-key 1 md.5 ?

<0-7> Encryption type (0 for not yet encrypted, 7 for proprietary)

Dublin(config-if)#ip ospf message-digest-key 1 md5 7 ?

LINE The OSPF password (key)

Dublin(config-if)#ip ospf message-digest-key 1 md5 7 asecret

c.	What is the OSPF password being used for md5 authentication?

d.	What encryption type is being used?

Step 8 Enable OSPF authentication in this area, area 0

Dublin(config-if)#router ospf 1

Dublin(config-router)#area 0 authentication message-digest

a.	Wait for a few seconds. Does the router generate any output?

b.	Enter the command show ip ospf neighbor.

c.	Are there any OSPF neighbors?

d.	Examine the routing table by entering show ip route.

e.	Are there any OSPF routes in the Dublin router routing table?

f.	Can the Dublin host ping the Washington host?

g.   Enter these configuration commands, one per line. End with CNTL/Z.

Washington#configure terminal

Washington(config)#interface serial 0

Washington (config-if) #ip ospf message-digest-key 1 md.5 7 asecret

Washington(config-if)#router ospf 1

Washington(config-router)#area 0 authentication message-digest

h.   Verify that there is an OSPF neighbor by entering show ip ospf neighbor command.

i.    Show the routing table by typing show ip route.

j.     Ping the Washington host from Dublin. If this was not successful troubleshoot as necessary.

Once the previous steps are completed, log off by typing exit and turn the router off. Then remove and store the cables and adapter.

3 -5          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.4	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted fora password, enter class. If that does not work, ask the instructor for assistance.

Router>enable

At the privileged EXEC mode, enter the command erase startup-config.

Router#erase startup-config

The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase of nvram: complete

Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload

The responding line prompt will be:

System configuration has been modified. Save? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Proceed with reload? [confirm]

Press Enter to confirm.

In the first line of the response will be:

Reload requested by console.

After the router has reloaded the line prompt will be:

Would you like to enter the initial configuration dialog? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Press RETURN to get started!

Press Enter.

Now the router is ready for the assigned lab to be performed.

4 -5          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.4	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

5-5

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.4

Copyright © 2003, Cisco Systems, Inc.

Cisco Systems

Lab 2.3.5 Configuring OSPF Timers

Area 0

Router 1

Router 2

| Router Designation | Router Name | Enable Secret Password | Enable, VTY, and Console Passwords | Routing Protocol | Network Statements |
| --- | --- | --- | --- | --- | --- |
| Router 1 | Sydney | class | cisco | OSPF | 192.168.1.0 |
| Router 2 | Rome | class | dsco | OSPF | 192.168.1.0 192.168.0.0 |

Router	IP Host FastEthernet 0 Interface Serial 0	Interface	 Loopback 0

Designation Table     Address/	Type         Address/Subnet Type Serial 1    Address/Subnet

Entry    Subnet Mask    Serial 0    Mask	 Mask

| Router 1 | Rome | 192.168.1.129/26 | DCE | 192.168.1.1/30 | NA | 192.168.31.11/32 |
| --- | --- | --- | --- | --- | --- | --- |
| Router 2 | Sydney | 192.168.0.1/24 | DTE | 192,1Bfl. 1.2/30 | NA | 192.168.31.22J32 |

Note: The IP Host Table Entry column contents indicate the narnB(s) of the other routers) in the IP host table.

Straight-through cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Setup an IP addressing scheme for OSPF area.

•	Configure and verify OSPF routing.

•	Modify OSPF interface timers to adjust efficiency of network.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

1 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.5

Copyright © 2003, Cisco Systems, Inc.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname, console, virtual terminal and enable passwords. Next configure the interfaces and IP hostnames according to the chart. Do not configure the routing protocol until specifically told to. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router Configuration with RIP".

Step 2 Save the configuration information from the privileged EXEC command mode

Sydney#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Rome#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Why save the running configuration to the startup configuration?

Step 3 Configure the hosts with the proper IP address, subnet mask, and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 9x/ME, check by using Start > Run > winipcfg. If running WindowsNT/2000/XP, check by using the ipconf ig command in a DOS window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using OSPF as the routing protocol.

Step 4 Verify connectivity

a.	On a router, ping the serial interface of the other router.

b.	What the ping successful?

c.   If the ping was not successful, troubleshoot the router configurations, until the ping is successful.

Step 5 Configure OSPF routing on both routers

a.	Configure OSPF routing on each router. Use OSPF process number 1 and ensure all networks are in area 0. Refer to the lab, "Configuring Loopback Interfaces" for review on configuring OSPF routing if necessary.

b.	Did the IOS version automatically add any lines under router OSPF 1 ?

c.	Show the routing table for the Sydney router.

Sydney#show ip route

d.	Are there any entries in the routing table?

Step 6 Test network connectivity

Ping the Sydney host from the Rome host. Was it successful?

If not troubleshoot as necessary.

2 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.5	Copyright © 2003, Cisco Systems, Inc.

Step 7 Observe OSPF traffic

a.	At the privileged EXEC mode type the command debug ip ospf events and observe the output.

b.	How frequently are Hello messages sent?

c.	Where are they coming from?

d.    Turn Off debugging by typing no  debug  ip  ospf  events or undebug  all.

Step 8 Show interface timer information

a.	Show the hello and dead interval timers on the Sydney router Ethernet and Serial interfaces by entering the command show ip ospf interface in privileged EXEC mode.

b.	Record the Hello and Dead Interval timers for these interfaces

c.	Hello Interval:

d.	Dead Interval:

e.	What is the purpose of the dead interval?

Step 9 Modify the OSPF timers

a.	Modify the Hello and Dead-Interval timers to smaller values to try to improve performance. On the Sydney router only enter the commands ip ospf hello-interval 5 and ip ospf dead-interval 20 for interface Serial 0.

Sydney(config)#interface Serial 0

Sydney(config-if)#ip ospf hello-interval 5

Sydney(config-if)#ip ospf dead-interval 20

b.	Wait fora minute and then enter the command show ip ospf neighbor.

c.	Are there any OSPF neighbors?

Step 10 Examine the routing table

a.	Examine the Sydney router routing table by entering show ip route.

b.	Are there any OSPF routes in the table?

c.   Can the Sydney Host ping the Rome host?

Step 11 Look at the OSPF data transmissions.

a.	Enter the command debug ip ospf events in privileged EXEC mode.

b.	Is there an issue that is identified?

c.	If there is, what is the issue?

Step 12 Check the Rome router routing table status.

a.	On the Rome router check the routing table by typing show ip route.

b.	Are there any OSPF routes in the table?

3 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.5	Copyright © 2003, Cisco Systems, Inc.

Step 13 Set the Rome router interval timers

a.	Match the timer values on the Rome Serial link with the Sydney router.

Rome(config)#interface serial 0

Rome(config-if)#ip ospf hello-interval 5

Rome (config-if)#ip ospf dead-interval 20

b.	Verify the OSPF neighbor by entering show ip ospf neighbor command.

c.	Show the routing table by typing show ip route.

d.	Are there OSPF routes in the table?

e.   Ping the Rome host from Sydney. If this was not successful troubleshoot the configurations.

Step 14 Reset the routers interval timers to the default values

Use the no form of the ip ospf hello-interval and the ip ospf dead-interval to reset the OSPF timers back to their default values.

Step 15 Verify the interval timers are returned to the default values

a.	Use the show ip ospf interface command to verify the timers are reset to their default values.

b.	Are the values back to the default?

c.    If not, repeat step 13 and verify again.

Once the previous steps are completed, logoff by typing exit and turn the router off. Then remove and store the cables and adapter.

4 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.5	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted fora password, enter class. If that does not work, ask the instructor for assistance.

Router>enable

At the privileged EXEC mode, enter the command erase startup-config.

Router#erase startup-config

The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase of nvram: complete

Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload

The responding line prompt will be:

System configuration has been modified. Save? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Proceed with reload? [confirm]

Press Enter to confirm.

In the first line of the response will be:

Reload requested by console.

After the router has reloaded the line prompt will be:

Would you like to enter the initial configuration dialog? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Press RETURN to get started!

Press Enter.

Now the router is ready for the assigned lab to be performed.

5 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.5	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

6 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.5

Copyright © 2003, Cisco Systems, Inc.

Cisco Systems

Lab 2.3.6 Propagating Default Routes in an OSPF Domain

Router 2 DTEl

| Router Router Enable Secret Enable, VTY, and Routing Network Loopback 0 Designation Name Password Console Passwords Protocol Statements Address/Subnet Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router 1 | Tokyo |  | cisco |  |  | OSPF |  |  | 192.168.1.0 192,168.31.11/32 |
| Router 2 | Madrid | class | Cisco |  |  | OSPF |  |  | 192.166.1-0 192.166 31.22/32 192.168.0.0 |
| Router IP Host FastEthernet 0 Interface Serial 0 Interface Serial 1 Designation Table Address/ Type Address/Subnet Type Serial 1 Address/Subnet Entry Subnet Mask Serial 0 Mask Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Router 1 | Madrid | 192.168.1.129/24 | DCE | 192.16S.1.1/30 |  |  | NA ¦ NA |  |  |
| Router 2 | Tokyo | 192.168.0.1/Z4 | DTE | 192.168.1.2/30 |  |  | DTE 200.20.20.2/30 |  |  |

Note. The IP Host Table Entry column contents indicate the name(s) of the other routerfs) in the IP host table.

Straight-trirough cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Setup an IP addressing scheme for OSPF area.

•	Configure and verify Open Shortest Path First (OSPF) routing.

•	Configure the OSPF network so that all hosts in OSPF area can connect to outside networks.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in the lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

1 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.6

Copyright © 2003, Cisco Systems, Inc.

Step 1 Configure the ISP router

Normally the ISP router would be configured by the Internet service provider (ISP). For the purpose of this lab, after erasing the old configuration, configure the ISP router (Router 3) this way by typing:

Router>enable

Router#configure terminal

Router(config)#hostname ISP

ISP(config)#line vty 0 4

ISP(config-line)#password cisco

ISP(configline)#login

ISP(config-line)tinterface serial 1

ISP(config-if)#ip address 200.20.20.1 255.255.255.252

ISP(config-if)#clock rate 64000

ISP (config-if)#no shutdown

ISP(config-if)#interface loopback 0

ISP(config-if)#ip address 138.25.6.33 255.255.255.255

ISP(configif)#exit

ISP(config)#ip route 192.168.1.0 255.255.255.0 200.20.20.2

ISP(config)#ip route 192.168.0.0 255.255.255.0 200.20.20.2

ISP(config)tend

ISP#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Building configuration...

[OK]

ISP#

Step 2 Configure the Area 0 OSPF routers

On the routers, enter the global configuration mode and configure the hostname, console, virtual terminal and enable passwords. Next configure the interfaces and IP hostnames according to the chart. Do not configure the routing protocol until specifically told to. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router Configuration with RIP".

Step 3 Save the configuration information from the privileged EXEC command mode

Tokyo#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Madrid#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Why save the running configuration to the startup configuration?

Step 4 Configure the hosts with the proper IP address, subnet mask and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 9x/ME, check by using Start > Run > winipcfg. If running Windows NT/2000/XP, check by using the ipconf ig command in a DOS window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using OSPF as the routing protocol.

2 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.6	Copyright © 2003, Cisco Systems, Inc.

Step 5 Verify connectivity

a.	Ping from the Madrid router to both the Tokyo and the ISP routers.

b.	Were the pings successful?

c.    If the ping was not successful, troubleshoot the router configurations, until the ping is successful.

Step 6 Configure OSPF routing on both area 0 routers

a.	Configure OSPF routing on each router. Use OSPF process number 1 and ensure all networks are in area 0. Refer to the lab "Configuring loopback interfaces" for review on configuring OSPF routing if necessary.

b.	Did the IOS version automatically add any lines under router OSPF 1 ?

c.	Show the routing table for the Tokyo router.

Tokyo#show ip route

d.	Are there any entries in the routing table?

Step 7 Test network connectivity

a.	Ping the Tokyo host from the Madrid host. Was it successful?

b.	If not troubleshoot as necessary.

Step 8 Observe OSPF traffic

a.	At privileged EXEC mode type the command debug ip ospf events and observe the output.

b.	Is there OSPF traffic?

C.    Turn Off debugging by typing no  debug  ip  ospf  events or undebug  all.

Step 9 Create a default route to the ISP

On the Madrid router only, type in a static default route through Serial 1 interface.

Madrid(config)#ip route 0.0.0.0 0.0.0.0 200.20.20.1

Step 10 Verify the default static route

a.	Verify the default static route by looking at the Madrid routing table.

b.	Is the default route in the routing table?

Step 11 Verify connectivity from the Madrid router

a.	Verify connectivity from the Madrid router by pinging the ISP Serial 1 interface from the Madrid router.

b.	Can the interface be pinged?

c.	Next, on the host attached to Madrid, open a Command Prompt and ping the serial 1 interface on the ISP router.

d.	Can the interface be pinged?

e.   This time, ping the loopback address of the ISP router, which represents the ISP connection to the Internet.

3 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.6	Copyright © 2003, Cisco Systems, Inc.

f.    Can the loopback interface be pinged?

g.   All of these pings should be successful. If they are not, troubleshoot the configurations on the host and the Madrid and ISP routers.

Step 12 Verify connectivity from the Tokyo router

a.	Verify the connection between the ISP and the Tokyo by pinging the serial 1 interface of the ISP router on the Tokyo router.

b.	Can the interface be pinged?

c.	If yes, why? If not, why not?

Step 13 Redistribute the static default route

a.	Propagate the gateway of last resort to the other routers in the OSPF domain. At the configure

router prompt on the Madrid router type default-information  originate.

Madrid(config-router)#default-information originate

b.	Is there now a default route on the Tokyo router?

c.   What is the address of the Gateway of last resort?

d.   There is an O*E2 entry in the routing table. What type of route it is?

e.	Can the ISP server address at 138.25.6.33 be pinged from both workstations?

f.	If no, troubleshoot both hosts and all three routers.

Once the previous steps are completed, log off by typing exit and turn the router off. Then remove and store the cables and adapter.

4 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.6	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted fora password, enter class. If that does not work, ask the instructor for assistance.

Router>enable

At the privileged EXEC mode, enter the command erase startup-config.

Router#erase startup-config

The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase of nvram: complete

Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload

The responding line prompt will be:

System configuration has been modified. Save? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Proceed with reload? [confirm]

Press Enter to confirm.

In the first line of the response will be:

Reload requested by console.

After the router has reloaded the line prompt will be:

Would you like to enter the initial configuration dialog? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Press RETURN to get started!

Press Enter.

Now the router is ready for the assigned lab to be performed.

5 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.6	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

6 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  2.3.6

Copyright © 2003, Cisco Systems, Inc.

Cisco Systems