Lab 3.2.1 Configuring EIGRP Routing

| Router Designation | Router Name | Enable Secret Password | Enable, VTY, and Console Passwords | Routing Protocol | Network Statements |
| --- | --- | --- | --- | --- | --- |
| Router 1 | Paris | class | cisco | EIGRP | 192.168.3,0 192.168-2 0 |
| Router 2 | Warsaw | class | cisco | EIGRP | 192.168.1.0 192.168.2.0 |

Router	IP Host        FastEthernet 0        Interface       Serial 0	 Loopback 0

Designation      Table	Address/	Type	Address/Subnet     Address/Subnet

Entry	Subnet Mask	Serial 0         Mask	 Mask

| Router 1 | Warsaw | 192.168.3.1/24 | DGE | 192.168.2.1/30 | 192.168.0.2/24 |
| --- | --- | --- | --- | --- | --- |
| Roj:er 2 | Paris | 192.169.1.1/24 | DTE | 192.168.2.2/30 |  |

Note: The IP Host Table Entry column contents indicate the name(s) of the other router(s) in the IP host table.

Straight-through cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Setup an IP addressing scheme for the network.

•	Configure and verify Enhanced Interior Gateway Routing Protocol (EIGRP) routing.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in the lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname as shown in the chart. Then configure the console, virtual terminal and enable passwords. Next configure the interfaces according to the chart. Finally, configure the IP hostnames. Do not configure the routing

1 -5

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.1

Copyright © 2003, Cisco Systems, Inc.

protocol until specifically told to. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router Configuration with RIP".

Step 2 Save the configuration information from the privileged EXEC command mode

Paris#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Step 3 Configure the hosts with the proper IP address, subnet mask and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 98, check by using Start > Run > winipcf g. If running Windows 2000, check by using the ipconf ig command in a DOS window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using EIGRP as the routing protocol.

Step 4 View the routers configuration and interface information

a.	At the privileged EXEC mode prompt type:

Paristshow running-config

b.	Using the show ip interface brief command, check the status of each interface.

c.	What is the state of the interfaces on each router? Paris:

FastEthernet 0:

Serial 0:

Warsaw:

FastEthernet 0:

Serial 0:

d.	Ping from one of the connected serial interfaces to the other.

e.	Was the ping successful?

f.    If the ping was not successful, troubleshoot the routers configuration, until the ping is successful.

Step 5 Configure EIGRP routing on router Paris

a.	Enable the EIGRP routing process on Paris, and configure the networks it will advertise. Use EIGRP autonomous system number 101.

Paris(config)#router eigrp 101 Paris(config-router)#network 192.168.3.0 Paris(config-router)#network 192.168.2.0 Paris(config-router)#network 192.168.0.0

Paris(configrouter)tend

b.	Show the routing table for the Paris router.

Paristshow  ip  route 2 -5          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.1	 Copyright © 2003, Cisco Systems, Inc.

c.	Are there any entries in the routing table?

d.	Why?

Step 6 Configure EIGRP routing on router Warsaw

a.	Enable the EIGRP routing process on Warsaw, and configure the networks it will advertise. Use EIGRP autonomous system number 101.

Warsaw(config)#router eigrp 101

Warsaw(config-router)#network 192.168.2.0

Warsaw(config-router)#network 192.168.1.0

Warsaw(configrouter)tend

b.	Show the routing table for the Warsaw router.

Warsaw#show ip route

Step 7 Test network connectivity

Ping the Paris host from the Warsaw host. Was it successful?

If not troubleshoot as necessary.

Once the previous steps are completed, log off by typing exit, and turn the router off. Then remove and store the cables and adapter.

3 -5          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.1	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted fora password, enter class. If that does not work, ask the instructor for assistance.

Router>enable

At the privileged EXEC mode, enter the command erase startup-config.

Router#erase startup-config

The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase of nvram: complete

Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload

The responding line prompt will be:

System configuration has been modified. Save? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Proceed with reload? [confirm]

Press Enter to confirm.

In the first line of the response will be:

Reload requested by console.

After the router has reloaded the line prompt will be:

Would you like to enter the initial configuration dialog? [yes/no]:

Type n and then press Enter. The responding line prompt will be:

Press RETURN to get started!

Press Enter.

Now the router is ready for the assigned lab to be performed.

4 -5          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.1	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

5-5

CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.1

Copyright © 2003, Cisco Systems, Inc.

Cisco Ststems

Lab 3.2.3 Verifying Basic EIGRP Configuration

Router 1

Router 2

| Router Designation | Router Name | Enable Secret Password | Enable, VTY, and Routing Console Passwords Protocol |  |  |  |  | Network Statements |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router 1 | Paris | class | CISCO |  |  | EIGRP | 192.168.3.0 192.168-2 0 |
| Router 2 | Warsaw | class | CISCO |  |  | EIGRP | 192.168.1.0 192.168.2.0 |
| Router Designation | IP Host Table Entry | FastEthernet 0 Address/ Subnet Mask | Interface Type Serial 0 | Serial 0 Address/Subnet Mask |  |  | LoDpback0 Address/Subnet Mask |
| Router 1 | Warsaw | 192.168.3.1/24 | DCE | 192.168.2.1*30 |  |  | 192.168.0.2/24 |
| Router 2 | Paris | 192.168.1.1/24 | DTE | 192.166.2.2/30 |  |  | No dcjijess |

Note; The IP Host Table Entry column contents indicate the name(s) of the other routers) in the IP host table.

Straight-through cable Serial cable

Console (Rollover) Crossover cable

Objective

•	Setup an IP addressing scheme for the network.

•	Configure and verify Enhanced Interior Gateway Routing Protocol (EIGRP) routing.

Background/Preparation

Cable a network similar to the one shown in the diagram. Any router that meets the interface requirements displayed on the above diagram may be used. For example, router series 800, 1600, 1700, 2500, and 2600 or any such combination can be used. Please refer to the chart at the end of the lab to correctly identify the interface identifiers to be used based on the equipment in the lab. The configuration output used in this lab is produced from 1721 series routers. Any other router used may produce slightly different output. Perform the following steps on each router unless specifically instructed otherwise.

Start a HyperTerminal session.

Note: Go to the erase and reload instructions at the end of this lab. Perform those steps on all routers in this lab assignment before continuing.

1 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.3

Copyright © 2003, Cisco Systems, Inc.

Step 1 Configure the routers

On the routers, enter the global configuration mode and configure the hostname as shown in the chart. Then configure the console, virtual terminal and enable passwords. Next configure the interfaces according to the chart. Finally, configure the IP hostnames. Do not configure the routing protocol until specifically told to. If there are any problems configuring the router basics, refer to the lab "Review of Basic Router configuration with RIP".

Step 2 Save the configuration information from the privileged EXEC command mode

Paris#copy running-config startup-config

Destination filename [startup-config]? [Enter]

Step 3 Configure the hosts with the proper IP address, subnet mask and default gateway

a.	Each workstation should be able to ping the attached router. Troubleshoot as necessary. Remember to assign a specific IP address and default gateway to the workstation. If running Windows 98, check by using Start > Run > winipcfg. If running Windows 2000, check by using the ipconf ig command in a DOS window.

b.	At this point the workstations will not be able to communicate with each other. The following steps will demonstrate the process required to get communication working using EIGRP as the routing protocol.

Step 4 View the routers configuration and interface information

a.	At the privileged EXEC mode prompt type:

Paris#show running-config

b.	Using the show ip interface brief command, check the status of each interface.

c.	What is the state of the interfaces on each router? Paris:

FastEthernet 0:

Serial 0:

Warsaw:

FastEthernet 0:

Serial 0:

d.	Ping from one of the connected serial interfaces to the other.

e.	What the ping successful?

f.    If the ping was not successful, troubleshoot the routers configuration until the ping is successful.

Step 5 Configure EIGRP routing on router Paris

a.   Enable the EIGRP routing process on Paris, and configure the networks it will advertise. Use EIGRP autonomous system number 101.

Paris(config)#router eigrp 101 Paris(config-router)#network 192.168.3.0 Paris(config-router)#network 192.168.2.0 Paris(config-router)#network 192.168.0.0

2 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.3

Copyright © 2003, Cisco Systems, Inc.

Paris(configrouter)#end

b.	Show the routing table for the Paris router.

Paris#show ip route

c.	Are there any entries in the routing table?

d.	Why?

Step 6 Configure EIGRP routing on router Warsaw

a.	Enable the EIGRP routing process on Warsaw, and configure the networks it will advertise. Use EIGRP autonomous system number 101.

Warsaw(config)#router eigrp 101

Warsaw(config-router)#network 192.168.2.0

Warsaw(config-router)#network 192.168.1.0

Warsaw(configrouter)#end

b.	Show the routing table for the Warsaw router. Warsaw#show  ip   route

c.	Are there any EIGRP entries in the routing table now?

d.   What is the address type in the EIGRP 192.168.2.0 route?

e.   What does the D mean in the first column of the routing table?

Step 7 Show EIGRP neighbors

a.	From the Paris router show any neighbors connected using the show ip eigrp neighbors command at the privileged EXEC mode prompt.

b.	Are any neighbors shown?

Step 8 Test network connectivity

a.	Ping the Paris host from the Warsaw host. Was it successful?

b.	If not troubleshoot as necessary.

Step 9 View the topology table

a.	To view the topology table, issue the show ip eigrp topology all-links command.

b.	How many routes are in passive mode?

c.	To view more specific information about a topology table entry, use an IP address with this command:

Paris#show ip eigrp topology 192.168.1.0

d.	Based on the output of this command, does it tell what external protocol originated this route to 192.168.2.0?

3 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.3	Copyright © 2003, Cisco Systems, Inc.

e.   Does it tell which router originated the route?

f.	Finally, use show commands to view key EIGRP statistics. On the Paris router, issue the show ip eigrp traffic command.

g.	How many hello packets has the Paris router received?

h.   How many has it sent?

Once the previous steps are completed, log off by typing exit, and turn the router off. Then remove and store the cables and adapter.

4 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.3	Copyright © 2003, Cisco Systems, Inc.

Erasing and reloading the router

Enter into the privileged EXEC mode by typing enable.

If prompted for a password, enter class. If that does not work, ask the instructor for assistance.

Router>enable At the privileged EXEC mode, enter the command erase startup-config.

Router#erase  startup-config The responding line prompt will be:

Erasing the nvram filesystem will remove all files! Continue? [confirm]

Press Enter to confirm. The response should be:

Erase  of  nvram:   complete Now at the privileged EXEC mode, enter the command reload.

Router(config)#reload The responding line prompt will be:

System  configuration  has   been  modified.   Save?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Proceed  with  reload?    [confirm] Press Enter to confirm. In the first line of the response will be:

Reload  requested by  console. After the router has reloaded the line prompt will be:

Would  you   like   to   enter   the   initial   configuration  dialog?    [yes/no]: Type n and then press Enter. The responding line prompt will be:

Press  RETURN  to  get   started! Press Enter. Now the router is ready for the assigned lab to be performed.

5 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.3	Copyright © 2003, Cisco Systems, Inc.

| Router Interface Summary |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Router Model | Ethernet Interface #1 | Ethernet Interface #2 | Serial Interface #1 | Serial Interface #2 |
| 800 (806) | Ethernet 0 (E0) | Ethernet 1 (E1) |  |  |
| 1600 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 1700 | FastEthernet 0 (FA0) | FastEthernet 1 (FA1) | Serial 0 (SO) | Serial 1 (S1) |
| 2500 | Ethernet 0 (E0) | Ethernet 1 (E1) | Serial 0 (SO) | Serial 1 (S1) |
| 2600 | FastEthernet 0/0 (FA0/0) | FastEthernet 0/1 (FA0/1) | Serial 0/0 (S0/0) | Serial 0/1 (S0/1) |
| In order to find out exactly how the router is configured, look at the interfaces. This will identify what type and how many interfaces the router has. There is no way to effectively list all of the combinations of configurations for each router class. What is provided are the identifiers for the possible combinations of interfaces in the device. This interface chart does not include any other type of interface even though a specific router may contain one. An example of this might be an ISDN BRI interface. The string in parenthesis is the legal abbreviation that can be used in IOS command to represent the interface. |  |  |  |  |  |  |  |  |

6 -6          CCNA 3: Switching Basics and Intermediate Routing v 3.1 -Lab  3.2.3

Copyright © 2003, Cisco Systems, Inc.

Cisco Ststems