# Module 7 – Spanning Tree Protocol

# Module overview

Three Diagrams

Diagram 1, Tabular

Spanning Tree Protocol

After completing this chapter, students will be able to perform tasks relating to the following:

- Redundant Topologies

- Spanning Tree Protocol

Diagram 2, Tabular

CCNA 640-801 Exam

This module will cover the following objectives for the CCNA 640-801 exam:

- Technology

• Describe the Spanning Tree Protocol

Diagram 3, Tabular

ICND 640-811 Exam

-  Technology

• Describe the Spanning Tree Protocol

# Module 7.1 – Redundant Topologies

# Section 7.1.1: Redundancy

Single Diagram

Diagram 1, Informative

Redundancy

Description – Displays two diagrams, the first contains a car with a ‘CRASH’ label and the comment “There is one car, can I drive to work?”. The second Diagram has two cars, only one of which has a ‘CRASH’ label the caption reads “There are two cars, can I drive to work?”

# Section 7.1.2: Redundant Topologies

Five Diagrams

Diagram 1, Road Map

Network of Roads Provides Redundant Paths

Description – Displays a common road map, showing streets, intersections, lights, shopping centres, parks etc.

Diagram 2, Relational

Topology with One Bridge Crossing River

Description – Displays a town centre and a suburban centre where the only joining roadway is a single bridge across a river

Diagram 3, Relational

Effect of Failure on Non-redundant Topology

Description: – Duplicates the previous image however the bridge has been removed. (effectively isolating the town centre and the suburban centre).

Diagram 4, Relational

Redundant Bridge Topology

Description – Displays our Town centre and Suburban centre on opposite sides of the river, in this scenario however there are two intact bridges spanning the river.

Diagram 5, Relational

Effects of Failure on Redundant Bridge Topology

Description – Displays the Town centre and Suburban area still have connectivity, even though a single bridge has been removed/damaged.

# Section 7.1.3: Redundant Switched Topologies

Three Diagram

Diagram 1, Relational

Simple Redundant Switched Topology

Description – Displays two network segments (segments ‘A’ and ‘B’) these segments are connected with two switches (Switch ‘A’ and Switch ‘B’). Segment 1 is attached to Router Y and Server/Host X. Segment 2 is unpopulated.

Diagram 2, Active Flow Diagram

Flood Frames for Unknown Addresses

Description – Displays a host system (source) sending a message to a destination system, since the intermediate switches do not have the MAC address of the destination they flood the message in an attempt to deliver the message.

Diagram 3,

Flood Broadcast Frames

Description – Displays a source host sending a broadcast frame which is passed on to all devices within the broadcast domain.

# Section 7.1.4: Broadcast Storms

Single Diagram

Diagram 1, Active Flow Diagram

Broadcast Storm

Description – Displays an example of a Broadcast storm which occurs as there are two switches between Segment A and Segment B, Both switches continue to flood the broadcast  frames received from the other switch – an endless loop.

Textbox – “This example is only showing broadcast frames being continuously forwarded using switches. This causes a broadcast storm because the frames will always be forwarded as long as a switch has a place to forward them to; all other frames are not shown to simplify the example”

# Section 7.1.5: Multiple Frame Transmissions

Single Diagram

Diagram 1, Relational

Multiple Frame Transmissions

Description – Displays a network with two segments (segment A, and Segment B). Segment B is unpopulated, Segment A is attached To a Server/Host X and a Router Y. The Server /Host X sends a Uni-cast frame to Router Y (Server/Host X still has the Router Y MAC address in its ARP cache). This uni-cast frame is received by the router through Segment A. If however Switch A  (Switch A and Switch B are both attached to both network segments) does not have the MAC address of the router then the frame is flooded through Segment B, and back to Segment A via Switch B (regardless of whether Switch B has the Router Y MAC address or not). This results in multiple copies of the same uni-cast frame being received by Router Y – This is clearly inefficient use of the network resources.

# Section 7.1.6: Media Access Control Database Instability

Single Diagram

Diagram 1, Relational

Media Access Control Database Instability

Description – Again displays our Segmented network (Segment A and Segment B). This network has two switches connecting the two Segments (One switch is redundant). In this scenario the switches learn that particular hosts/servers/devices are on the incorrect network segments due to the initial frames being flooded out the alternate segment (e.g. a device on Segment A is thought to be on Segment B by Switch B as it received the forwarded frame from Segment B via Switch A).

# Module 7.2 – Spanning Tree Protocol

# Section 7.2.1: Redundant Topology and Spanning Tree

Two Diagrams

Diagram 1, Relational

Using Bridging Loops for Redundancy

Description – Displays a Wiring Closet (Consisting of two switches, Cat-6, and Cat-7), Also displays a Backbone network (Comprising 4 switches Cat-1 through Cat-4), lastly a Server Farm is shown (Containing a single switch Cat-5). Left to right new see the Wiring closet followed by the Backbone and then the Server Farm. Cat-6 and Cat-7 (In the Wiring Closet) are both connected to Cat-1 and Cat-3 in the Backbone (there is no direct connection between Cat-6 and Cat-7). The four Switches in the Backbone all have direct connections to the other backbone switches. Cat-5 (Switch in the Server Farm) has only two direct connections to the Backbone, Cat-5 connects to both Cat-2 and Cat-4 in the Backbone.

Diagram 2, Relational

Broadcast Storm

Description - See 7.1.4 Diagram 1

# Section 7.2.2: Spanning-Tree Protocol

Three Diagrams

Diagram 1, Relational

Spanning Tree Protocol

Description – Displays two switches in parallel to provide redundancy, but causes other conflicts, to resolve these issues the STP (Spanning Tree Protocol) effectively blocks one of the switch ports. STP provides a logical loop free, redundant network topology by placing certain ports in the blocking state.

Diagram 2, Tabular

Spanning-Tree Link Costs

Description – Displays a Table matching Link Speed and Costs

Link Speed: 10Gbps

Cost (Revised IEEE): 2

Cost (Previous IEEE): 1

Link Speed: 1Gbps

Cost (Revised IEEE): 4

Cost (Previous IEEE): 1

Link Speed: 100 Mbps

Cost (Revised IEEE): 19

Cost (Previous IEEE): 10

Link Speed: 10 Mbps

Cost (Revised IEEE): 100

Cost (Previous IEEE): 100

Diagram 3, Relational

A Spanning Tree

Description - Displays a three switch/three segment network. The links are labels with appropriate costs dependant upon link type, and one of the ports is shown to have been blocked by the STP to create a logical loop free network.

# Section 7.2.3: Spanning –Tree Operation

Single Diagram

Diagram 1, Relational

A Spanning Tree

Description – Displays our now familiar segmented network (Segment A, and Segment B) connected by two switches (Switch A and Switch B) one of which is redundant. The link speed of Segment A is indicated to be 100 Mbps, whereas Segment B is shown as 10 Mbps. Switch A has been selected as the Root Bridge, since Switch B is the redundant switch and given that Segment B is slower than Segment A the Switch B port to Segment B is the port that is blocked by the STP.

# Section 7.2.4: Selecting the Root Bridge

Four Diagrams

Diagram 1, List

Bridge Protocol Data Unit

Description – Lists the components of the BPDU and gives a brief query of each

Component: Root ID

Description: Who is the root bridge ?

Component: Root Path Cost

Description: How far away is the root bridge ?

Component: Sender BID

Description: What is the BID of the bridge that sent this BPDU?

Component: Port ID

Description: What port on the sending bridge did this BPDU come from?

Diagram 2, Informative

Bridge IDs

Description – Displays the BID, The BID is shown as 8 bytes total length, the first two bytes determine the Range (0-65535) with a default value of 32,768. The last 6 bytes are the MAC address from the Backplane/supervisor.

Text – “-Bridge ID (BID) is used to identify each bridge/switch

The BID is used in determining the centre of the network, in respect to the STP, Known as the Root Bridge 	“

Diagram 3, Informative

Inferior BPDU

Description – Displays the components and contents for an inferior BPDU of a switch with MAC address AAAA.AAAA.AAAA, and termed Cat-A.

Component: Root BID

Contents: 327768,AAAA.AAAA.AAAA

Component: Root Path Cost

Contents: 0

Component: Sender BID

Contents: 327768,AAAA.AAAA.AAAA

Component: Port ID

Contents: Sending Port ID e.g. 13

Diagram 4, Relational

Electing the Root Bridge

Description – Displays a three switch, 3 segment network. STP has blocked a port on Switch C to create a loop-free network, The Root Bridge, Bridge-A has two Designated ports, Switch B has a Root Port (Port connecting to the Root Bridge) and a Designated Port (Forwarding). Switch C has a Root Port only (One port is blocked to prevent a loop).

# Section 7.2.5: Stages of Spanning-Tree Port States

Single Diagram

Diagram 1, Flow Chart

Spanning-Tree Port States

Description – Shows a Spanning Tree Port state transitions, Disabled to Blocking to Listening to Learning to Forwarding. If a new port comes up the port goes into Blocking mode until the switch can determine is it is a Root or Designated Port.

# Section 7.2.6: Spanning Tree Recalculation

Two Diagrams

Diagram 1, Active Flow Diagram

Initial Spanned Network

Description – Displays a converged network, comprising three switches, Cat-A Switch is the Root Bridge. The Cat-A switch has two connections to Cat-C Switch, one of which is blocked. Cat-A Switch also has two connections to Cat-B Switch, again one of these links is blocked.

Diagram 2, Active Flow Diagram

Spanned Network after Port 1/1 Cat-B Fails

Description – Displays an image of the previous diagram, however the active link between Cat-A, and Cat-B has failed. All ports go into the Blocked state and the new Spanning tree is determined as the Switches proceed through the five operational states.

# Section 7.2.7: Rapid Spanning Tree Protocol

Two Diagrams

Diagram 1, Relational

Rapid Spanning Tree Port Designations

Description – Displays a three Switch, three segment network, (Comprising Switches X, Y, and Z). All links are 100 Mbps. Switch Z is the Root Port. The essential difference between this diagram and those prior is the distinction of the Blocking ports becoming ‘Alternate’ ports.

Diagram 2, Relational

Rapid Spanning Tree Link Type Designations

Description – Displays a detailed network. The essence of this picture is to emphasise that The Point-to-Point links and the Edge-Type links can go straight to Forwarding mode.

# Summary

Single Diagram

Diagram 1, Listing

Summary

Description – Lists the following key points

- Redundancy in a network is extremely important because redundancy allows networks to be fault tolerant.

- Redundant Topologies protect against network downtime due to a failure of a single link, port, or network device

- The Spanning-Tree-Protocol is used in switched networks to create a loop free logical topology from a physical topology that ahs loops.

- The four states of a Spanning Tree Port are:

  - Blocking

  - Listening

  - Learning

  - Forwarding