This file explains how to run the application code on either the DSK debugger

(DSK5D.EXE) or the DSK Loader (DSK5L.EXE).

The TMS320C5x DSK software package contains applications code for testing the

DSK memory, setting up the AIC and testing out the host/target communications

interface. The three application programs are listed below:

I. SELFTEST.OUT   ==> Automatic test file for the DSK board.

II. FUNC.DSK       ==> File allow you to check the AIC

III. DSK_SPEC.DSK   ==> Allow you to display the spectrum on the OSC.

I. SELFTEST.OUT

============

1. Connect the DSK to the RS232 properly and copy SELFTEST.OUT under

the working directory.

2. Run the test file by exexcuting the DSK loader with the following

parameters:

DSK5L -A -C1          (If using comport 1)

DSK5L -A -C2          (If using comport 2)

The parameters passed to DSK5L tell the program to automatically

load the file SELFTEST.OUT and execute. First DSK5L will fill the

DSK internal memory with a specified pattern then read them out to

check the DSK memory. After memory test the SELFTEST was loaded and

used to test the AIC. If any test failed or the test is finished DSK5L

will stop execution.

3. A good DSK will respond with the following messages

DSK-RS232 LOOP                  PASSED

Check Data Memory B0            0x0100  0x0300

>>>>>>>>                        PASSED

Check Data Memory B1            0x0300  0x0500

>>>>>>>>                        PASSED

Check Data Memory SARAM         0x0a00  0x2c00

>>>>>>>>                        PASSED

AIC test                        Send -> 0xb000  Receive -> 0x8000

PASSED!

Your DSK already passed the test !!!

4. If any of the tests fail a  'TEST FAILED' message will be generated

and DSK5L will stop execution.  If the RS232 link cannot be established,

DSK5L will likely not load and SELFTEST will not be attempted.  If there

are problems, go to the section "DSK5L COMMUNICATIONS AND SUB-FEATURES"

in this document.

II. FUNC.DSK

========

1. Connect the DSK to the RS232 properly. Connect a small, low power speaker

to the DSK output RCA jack.

2. Run the DSK debugger program by typing the following:

DSK5D

If the debugger did not build the communication link, try to change the

comport or reduce the baud rate. Be sure the PC's baud rate is similar to

the baud rate of the DSK.

3. After the debugger screen appears, load the FUNC.DSK file by hitting the

"L" key for LOAD, followed by "D" for DSK-format file in the sub-menu.

Next type "FUNC.DSK" at the prompt to load the file, and hit enter. The

screen will show the file sections being loaded. When the file has

finished loading, the debugger will ask you to 'press any key to

continue'.

4. The default setup for FUNC.DSK is to generate random noise. Press "X"

(EXECUTE) followed by "G" (GO) you will hear the random noise from

the audio output port.  Changing the value at memory location 0xF0D,

will generate a sinusoid output. This is done by hitting "F" for FILL,

followed by "D" for DATA memory. At the FILL MEMORY sub-menu, type 0F0D

for address, 1 for length <hit enter> and 1 for pattern <hit enter>. Now

change PC to starting address(0xA00) by pressing "M" to MODIFY and then

"R" for REGISTER. At the prompt type 'PC=0a00' and enter. Hit enter again

to return back to assembly screen. You have just modified the PC register

contents to address 0xa00. Go ahead and run the program again (XG). This

time you will hear the sinewave output.

5.  Changing the sampling rate will generate a different frequency of sinwave.

To do this you need to change the contents on memory 0xF00 & 0xF01. These

two position contain the value of the TA and RA registers respecively on

AIC. (For more information on how the AIC determines the sampling rate

please refer to Appendix B of the DSK User's Guide). After modifying the

TA and RA registers, restart the program from the beginning in order to

re-initalize the AIC with the new TA and RA values.

III. DSK_SPEC.DSK

============

1. Connect the DSK to the RS232 properly and copy DSK_SPEC.DSK to the

working directory.

2. Connect an oscilloscope to the output jack and a microphone to the input

jack of the DSK.

3. Run the test file by typing the following

DSK5L DSK_SPEC.DSK -C1          (If using comport 1)

DSK5L DSK_SPEC.DSK -C2          (If using comport 2)

The parameters passed to DSK5L instruct it to load the file named

DSK_SPEC.DSK and execute.

4. Turn on the oscilloscope power. Adjust the trigger and SEC/DIV until you

are able to see the spectrum of the DSK input signal.