*********************************************************

*                  >>>>>>> NOTE <<<<<<<                 *

*                                                       *

*             THIS IS A PRELIMINARY DOCUMENT            *

*     THE DSP STARTERS KIT IS NOT YET PRODUCTIZED       *

*                                                       *

*********************************************************

C5X DSP STARTERS KIT

( D S K 5 )

USERS GUIDE AND TECHNICAL REFERENCE

DSK HARDWARE AND SOFTWARE LICENSE

---------------------------------

Hardware

Your unmodified DSK circuit board has been carefully tested and

inspected in the factory for functionality.  Be sure to check the

board yourself BEFORE making any changes!  If you do make changes, this

will invalidate any kind of warranty, and we will not accept the board

back for repair.

Software

DSK5D.EXE (tm) TMS320C50 RS232 Monitor and Debugger   Version 1.xx

Copyright 1993 Texas Instruments, Inc.  All rights reserved.

DSK5A.EXE (tm) TMS320C5x DSP Designers Kit Assembler Version 1.xx

Copyright 1993 Texas Instruments, Inc.  All rights reserved.

DSK5L.EXE (tm) TMS320C5x DSP Designers Kit Loader Version 1.xx

Copyright 1993 Texas Instruments, Inc.  All rights reserved.

Introduction to C50 DSP Designers Kit

---------------------------------

OVERVIEW

Your C50 DSK development kit is designed to be operated from an IBM PC/AT

or compatible.  The user interface is through an RS-232 serial communications

link.  Provisions have been made in the hardware specification making it

possible to upgrade the DSK module with extra memory and features of your

own design.

KIT CONTENTS: Your kit should contain the following materials.

- DSK circuit board module

> TMS320C50 DSP processor

> TLC32040 Analog Interface Circuit (AIC)

> RS-232 interface chips

> AC/DC Power supply circuitry

- Floppy disk containing

> DSK5D.EXE    - The debugger interface for the DSK module

> DSK5A.EXE    - A TMS320C5x assembler

> DSK5L.EXE    - A DSK loader program

> ?????        - Check the subdirectories!  New applications will be

added as they become available!

- DSK Users Guide and Technical Reference

=======================================================================

WHAT YOU WILL NEED TO PROVIDE

-----------------------------

Like C26 DSK, your C50 DSK circuit board and software has been designed

to be operated from an IBM PC/AT running DOS 4.01 or higher. Your

computer must have an RS-232 asynchronous port installed to run the DSK.

Also, a suitable 9VAC (UL Class II) wall mount transformer with a 2.1mm

pin diameter power jack.  The outer diameter is 5.50mm.  A chart showing

the DSK connector and corresponding plugs is given below.

+----------+--------+--------+

|          | DSK    | XFMR   |

|          | CONN   | PLUG   |

+----------+--------+--------+

| PIN   ID | 2.00mm | 2.10mm |

| SHAFT OD | 5.55mm | 5.50mm |

+----------+--------+--------+

RS232 PORT AND CABLES

---------------------

To run the DSK you will need to have an RS-232 port available.  If you

do not, or the one you have is currently being used (possibly a mouse), you

may need to purchase an Asynchronous communications for your PC or possibly

an RS-232 upgrade kit.  Consult your PC manual for more details.

You will also need to purchase an RS-232 cable to connect to the DSK module.

To save space the DSK was designed with a DB9 RS-232 connection.  This

connector style is very popular with newer PC's and laptop computers.  If

you do not have the neccessary cables, they can be purchased from a local

computer store.  If you are making your own cable, the DSK module connections

as shown below.

DSK CIRCUIT BOARD

..........................................

:         RS232 LINE    C50 DSP          :

-----+             :    / |    BUFFERS     +----+           :

|TX   (XMIT)  :3 /   |  |\            |    |           :

PC/AT|---------------|    |--|  >o---------|BIO |  +-----+  :

HOST |TR   (RCV)   :2|   R|  |/  /|        |    |--|     |--- AIN

|---------------| D S|---o<  |--------|XF  |--| TLC |  :

ASYNC|DTR  (/RS)   :4| B 2|  |\  \| |\     |    |--|32040|  :

SER  |---------------| 9 3|--|  >o--|  >o--|/RS |--| AIC |--- AOUT

PORT |GND          :5|   2|  |/     |/     |    |--|     |  :

|---------------|    |----------------|GND |  +-----+  :

|             :  \   |                |    |           :

-----+             :    \ |                +----+           :

:                       +-----------+    :

:       +12(unreg) -----| Half Wave |    :

:               +5 -----| rectify   |----- 9 VAC

:               -5 -----| +/-5V reg |    :

:       -12(unreg) -----|           |    :

:                       +-----------+    :

..........................................

POWER SUPPLY

------------

The C50 DSK uses the same power supply design as the C26 DSK. You will need

a 9 VAC (>250 mA) supply to run the DSK.  The DSK module accepts a 2.1mm power

jack, common to many wall mount AC transformers.  A low current UL listed

transformer is recommended for the DSK module since they are designed to

withstand most power faults for a brief period of time.  If you do have

problems, immediately unplug the transformer from the wall and check that you

have everything set up right. In the United States a suitable transformer (9

VAC, 1.3A) can be aquired from Radio Shack (Cat. No. 273-1611A).  However, if

you have a early revision board you will also want to replace the 2.1mm jack on

this transformer.

If you decide to use the DSK's on board power supply and regulators for

external circuits, be sure to keep the loading to less than about 50 mA.

Regulation for the DSK is minimal and can not handle a lot of additional

current.

The DSK module can also be run from a suitable +/-7 to +/-12 VDC supply.

In this case the minimum voltage is set by the lower regulation limit of

the onboard +/-5 V regulators and the upper limit by the maximum allowable

RS232 level.  To do this, power is directly applied to the double row header

where all signals have been made available.  Note: Be careful to connect

the power supply correctly since the DSK is not warrentied after modifications

have been made.  Failure to do this can also cause significant damage to

the DSK module.

SETTING UP THE DSK SOFTWARE

---------------------------

It is strongly recommended that you first copy the contents of the DSK

distribution disk to a safe place for backup.  This can be done by using the

DOS DISKCOPY utility.  It is not recommended that you use the DSK from the

distribution diskette.

Next, we need to execute the install batch file (DSKINSTL.BAT) to install the

software to the C:\DSK sub-directory.  Since the DSK5D debugger is also

compatible with TI's COFF assemblers and linkers you can also copy everything

to that directory to keep things simple.  Next you should modify your PC's path

statement to make the DSK tools available from other directories.

The DSK software you will need primarily consists of three executable files,

DSK5D.EXE, DSK5A.EXE, and DSK5L.EXE.  The first two files are the debugger and

assembler respectively.  The third one is loader program. The other files

contained in the distribution disk are for examples and development support.

When the DSK5D is first started it also creates some configuration and

parameter file (DSK5D.CFG and DSK5D.PRM).  They can be modified to customize

the colors that the debugger uses.  If something goes wrong, delete the

old ones and start over.

=============================================================================

DSK5D

-----

INTRODUCTION

DSK5D is a debugger and user interface designed to be used with the EPROM

Bootload option of the TMS320C50 chip.  Features of DSK5D include an easy to

use windowed display of program, data, internal registers and status.  The

user interface is capable of loading and executing user code with single

step, breakpoint and run-time halt capabilities.  The full featured DSP code

which is responsible for the host communications has been kept to a very

compact 314 words in program SARAM making it possible to run reasonably

complicated code completely on-chip while still operating the debugger.

INPUT FILES FOR DSK5D

Valid code formats for DSK5D include both DSK output files from the DSK5A

assembler and COFF format files from the TMS320 DSP Fixed Point Assembler.

If you need features not found in the DSK5A assembler, contact your local

TI sales representitive for purchasing information on the COFF assembler.

HOW DSK5D WORKS

DSK5D works in conjunction with the EPROM bootload option of the

TMS320C50.  When the 'C50 is reset, the on-chip ROM bootloader drives the BR\

pin low and starts to load the kernel program from the OTP on the DSK. After

loading, the program starts from 0x800 on SARAM which is configured as program

and data memory.

The DSK uses BIO and XF to communicate with RS232. When the BIO line goes

low, this indicates that an RS232 transmission start bit has been sent by the

host.  Since the baud rate is unknown, the PC sends 0x80 data through RS232

and the 'C50 begins counting the number of elapsed CPU cycles until the BIO

line goes high again.  The baud rate is set by the width of the start bit plus

7 data bit, then divide by 8.

The bootloader loads code into on-chip SARAM.  In the case of the DSK a

debugging and communications kernal is loaded which is capable of bi-

directional RS232 communications using XF and BIO as well as debugging using

the TRAP mneumonic and INT2 (incoming RS232 message).  When a debug interrupt

occurs, the DSK kernal also saves and restores the internal registers.  This

should not concern a DSK user as long as the kernal is not corrupted.  If it is

corrupted, the DSK5D (and the DSP) will not operate properly.

>>>>>>  IMPORTANT <<<<<<

Your DSK module uses the INT2 line tied to the BIO input to indicate that

there is activity on the RS232 receive line.  If you turn off the global

interrupt enable bit or unmask INT2 the runtime halt feature will not work.

Likewise, if you rewrite the vector table for either TRAP or INT2 the debugger

kernal will fail to operate correctly.

RESERVED FOR DSK5D

> All of RAM block B2                 CPU register storage

> First 384 words of SARAM            debugger communications kernal

> SARAM configured as

program & data RAM                            ""

> C50 must be in uC mode              RS232 receive is in ROM

> RESET

> INT2                                Halts running CPU

> TRAP opcode                         Breakpoints and single step

INTERRUPT VECTOR TABLE

The interrupt vectors of the DSK have been allocated to start at 0x800. That

means the IPTR value in the PMST register is 1, and You SHOULD NOT MODIFY THE

IPTR VALUE. The DSK5D program will restore the RESET, INT2, TRAP interrupt

vectors. Therefore, you should not worry about overwriting those vectors. The

kernel program starts from 0x840 -DO NOT LET YOUR INTERRUPT VECTORS OVERWRITE

THE KERNEL PROGRAM. Your program should start after 0x980. We recommend you

start from 0xa00. The example code below shows where the DSK interrupt vector

table and kernal reside.

.ps     0800h

RESET:  .space  2*16          ;00; Reset vector reserved for Kernel program

INT1:   B       ISR1          ;02; external INT1

INT2:   .space  2*16          ;04; INT2 reserved for Kernal program

INT3:   B       ISR3          ;06; external INT3

TINT:   B       TIMER         ;08; Timer interrupt

RINT:   B       RECEIVE       ;0A; Serial port receive interrupt

XINT:   B       TRANX         ;0C; Serial port transmit interrupt

TRNT:   B       TREC          ;0E; TDM serial port receive interrupt

TXNT:   B       TTRANX        ;10; TDM serial port transmit interrupt

INT4:   B       ISR4          ;12; external INT4

;--------------------------------------------------------------

; KERNEL PROGRAM (00840h - 00980h)

;--------------------------------------------------------------

.ps    00a00h        ;Your code can start here

.entry               ;

START:   mar    *,AR0         ;

lar    AR0,#0        ;

lacl   #03fh         ;Turn on all interrupts

ldp    #0            ;

sacl   04h           ;

SELF:    B      SELF          ;

;---------------------------------------------------------------

; Simple interrupt processing (quick return)

ISR1:                         ;

ISR3:                         ;

ISR4:                         ;

TIMER:                        ;

RECEIVE:                      ;

TRANX:  ldp    #0             ;recover the ARP from ST1

.

.

.

rete                  ;

.end                  ;

DSK5D COMMANDS AND HELP MENU

For quick reference, a list of commands can be viewed from within DSK5D by

hitting 'H' (Help) while at the command line.  This list can also be sent to

a file while in the Help menu by hitting the 'F' (File) key. The following is

the output listing:

D S P    S T A R T E R     K I T    D E B U G G E R

����������������������������������������������������������������������

Usage of the program: dskd [[-|\][options]

Options: Please note that options are not case sensative

��������

H      : Displays this screen

Bxxxxx : Selects the baudrates: 4800, 9600, 19200[default], 38400, or 57600.

COMx   : comport: x=1 [default] or x=2. Optional 'C1' or 'C2' can be used to

select commport 1 and commport 2 respectively. (commports 3 & 4 are

not supported)

Exxxx  : defines the entry point, where xxxx is the address in hexidecimal.

I      : selects logic level for DTR->Reset (default->inverse)

L      : selects the EGA/VGA screen length (43 or 50)

S      : selects the default screen length (25)

Function Key definitions

������������������������

F1     Help Information

F2     Print Screen to 'screen.srn'

F3     Display Directory

F4     unused

F5     execute to breakpoint --> same as XG

F6     unused

F7     unused

F8     singlestep --> same as F10, XS, blank, return

F9     unused

F10    execute over function --> same XF

F11    Reverse Assembler window

sF1    Reverse Assembler window

F12    Trace ON/OFF

sF2    Trace ON/OFF

BA     Breakpoint add

BD     Breakpoint delete

BE     Breakpoint enable/disable

BL     Breakpoint list

C..    Copy/Move

CDD    Copy Data block to another Data area

CDP    Copy Data block to Program area

CPD    Copy Program block to Data area

CPP    Copy Program block to another Program area

DD     Display Data memory

DB     Display Breakpoints

DF.    Display Format: see subcommands

DFB    Display Data in format big (expo=16,mantissa=32/Q30)

DFD    Display Data in format double

DFE    Display Data in format short float (exp=16,mant=16/Q14)

DFF    Display Data in format float

DFI    Display Data in format integer

DFL    Display Data in format long

DFO    Display Data in format octal

DFP    Display Data in format packed string

DFQ    Display Data in format signed Q15

DFS    Display Data in format string

DFU    Display Data in format unsigned integer

DFT    Display Data in format long float (exp=32,mant=64/Q62)

DFX    Display Data in format hexadecimal

DM.    Display Memory window: see subcommands

DMA    Display Memory window: set Address

DMB    Display Memory Format: big (expo=16,mantissa=32/Q30)

DMD    Display Memory Format: double (IEEE)

DME    Display Memory Format: short float (exp=16,mant=16/Q14)

DMF    Display Memory Format: float (IEEE)

DMI    Display Memory Format: integer

DML    Display Memory Format: long

DMO    Display Memory Format: octal

DMQ    Display Memory Format: Q15

DMU    Display Memory Format: unsigned int

DMT    Display Memory Format: long float (exp=32,mant=64/Q62)

DMX    Display Memory Format: to hexadecimal

DP     Display Program memory

DS     Display Status register information

DV     Display Version

FD     Fill Data Memory with a value

FP     Fill Program Memory with a value

Help   Help display

I      Initialize to the entry point of the program

L..    Load data (program, [formatted] data from a file)

LC     Load Coff file: common object file format

LD     Load DSK file: DSK file format

LP     Load Format Program: 4 hex digits per line

LF.    Load Format

LFB    Load Format Bfloat: 1 big float per line

LFD    Load Format Double: 1 IEEE double number per line

LFE    Load Format Efloat: 1 special float per line

LFI    Load Format integer: 1 int per line

LFL    Load Format long: 8 hex digits per line

LFF    Load Format Float: 1 IEEE float number per line

LFP    Load Format packed charter string

LFQ    Load Format Q15: one Q15 number per line

LFS    Load Format Char string: 1 char per word

LFT    Load Format Tfloat: 1 long float per line

LFU    Load Format unsigned: 1 unsigned int per line

LFX    Load Format Hex: 4 hex digits per line

MR     Modify 320C50 Register: PC,ACCU,ARx, etc.

MP     Modify Program Memory

MD     Modify Data Memory

Q      Quit (or ctrl Q)

P      load Program counter: done from ModifyRegister display

R      Reset the 320c50

S..    Save (program, [formatted] data to a file)

SD     Save Data memory to file

SF     Save Format

SP     Save Program memory to file: 4 hex digits per line

SFB    Save Format Bfloat: 1 big float per line

SFI    Save Format integer: 1 int per line

SFD    Save Format Double: 1 IEEE double number per line

SFE    Save Format Efloat: 1 special float per line

SFF    Save Format Float: 1 IEEE float number per line

SFL    Save Format long: 8 hex digits per line

SFP    Save Format Packed character string

SFQ    Save Format Q15: 1 Q15 number per line

SFS    Save Format character String: 1 char per word

SFT    Save Format Tfloat: 1 long float per line

SFU    Save Format unsigned: 1 unsigned int per line

SFX    Save Format Hex: 4 hex digits per line

V      Verify Flag ON/OFF - default=ON for(Load ...)

W..    Watch Window definition

WA     Watch window: Address definition

WD     Watch window: Delete watch number

WM     Watch window: Modify address of watch

WF     Watch window Format: see subcommands

WFB    Watch window Format: big (exp=16,mant=32/Q30)

WFD    Watch window Format: double

WFE    Watch window Format: short float (exp=16,mant=16/Q14)

WFF    Watch window Format: float

WFI    Watch window Format: integer

WFP    Watch window Format: packed string

WFQ    Watch window Format: Q15 (sign . fraction)

WFS    Watch window Format: string

WFU    Watch window Format: unsigned integer

WFT    Watch window Format: long (exp=32,mant=64/Q62)

XA     eXecute and go to Address: define a destination Adress

XC     eXecute with calling a function (includes stack handling)

XF     eXecute a function: CALL or CALA

XG     eXecute Go to Breakpoint

XL     eXecute and Link: branches like a 'call' to an address

XQ     eXecute and Quit the debugger

XN     eXecute 'n' single steps

XR     eXecute and Free Run (ignore switching XF)

XS     eXecute one single step

see: F8, F10, blank, return

Floating Point formats:

�����������������������

.Float   32 bit IEEE standardized floating point format

.Double  64 bit IEEE standardized double floating point format

.Bfloat  'big float' / 16 bit exponent + 32 bit mantissa

exponent: 2's complement / signed mantissa = Q30

.Tfloat  'long float' / 32 bit exponent + 64 bit mantissa

exponent: 2's complement / signed mantissa = Q62

.Efloat  'enhanced float' / 16 bit exponent + 16 bit mantissa

exponent: 2's complement / signed mantissa = Q14

Register Definitions:

�������������������������������������������

PC    Program counter: 16 bit

ACCU  Accumulator: 32 bit with C (carry) in ST1

ACCB  Accumulator Buffer: 32 bit to temporarily store ACCU

PREG  Product Register: 32 bit for 16* 16 bit multiplication

TRG0  Temporary Multiplicand: 16 bit for multipliction

and special instructions

TRG1  Temporary Register 1: 5 bit for dynamic shift

TRG2  Temporary Register 2: 4 bit for bit pointer in bit test

ARi   Auxiliary Register: 16 bit with i=0..7 used as

counter and pointer

ST0   Status Register 0: 16 bit

ST1   Status Register 1: 16 bit

PMST  Status Register: 16 bit

STCKi Stack Register: 16 bit with n= 0..7 used as

Hardware Stack.

Please note that the debugger uses one level stack

for its own purpose!!

DRR   Data receive register at address 0:

16 bit for the serial port

DXR   Data transmit register at address 1:

16 bit for the serial port

TIM   Timer register at address 24: 16 bit

PRD   Period register at address 25: 16 bit

IMSK  Interrupt mask register at address 4:

6 bits for masking 6 interrupts

GREG  Global register at address 5:

8 bits to define data memory as global

SPC   Serial Port Control Register at address 22

ARP   ST0: Auxiliary register pointer: 3 bits

INTM  ST0: Interrupt mode (global int enable) : 1 bit

DP    ST0: Data Page pointer: 9 bits

ARB   ST1: Auxiliary register pointer buffer: 3 bits

CNF   ST1: internal Prog/Data configuration: 2 bits

PM    ST1: Preg to accu shift mode: 2 bits

C     ST1: Carry bit: 1 bit

TC    ST1: Test/Control: 1 bit

HM    ST1: Holdmode selection: 1 bit

TXM   SPC: FSX mode bit: 1 bit

FO    SPC: serial port control (8/16 bit mode): 1 bit

Please note that following is reserved

��������������������������������������

1) IMR=4 (INT2) and EINT are set from the debugger to enable

halting of a running program: Please do not overwrite

the bit IMR=4 and use logical instruction to set the

appropriate interrupt mask! Do not change the locations

0x804 and 0x805, which are reserved for INT2!

If another IMR bit than 4 is set this Interrupt is

executed also during Single Steps!

If an breakpoint is set on an interrupt vector (0x802...)

and then it is executed with Single Step it will 'nest'

interrupts because EINT is automatically executed!

2) Block B2 (data memory 0x60 - 0x7f) is reserved for

debugger use! Don't overwrite this locations!

3) The debugger kernel is located from 0x840-0x980

in SARAM configured as Program & Data memory! Please

DO NOT modify RAM & OVLY bits.

4) The instruction TRAP and the memory location 0x822

(trap vector) are reserved for the debugger

Please note also following

�������������������������

The debugger allocate the Interrupt Vectors at 0x800 and

higher by set IPTR=1! Please DO NOT modified the IPTR value

Executing User Interrupts (please note that INT2 is reserved)

requires that additional Vectors have to be placed into the

address space 0x800-0x83f as following:

0x802-0x803   -  INT 1

0x804-0x805   -  INT 2 - used from debugger

0x806-0x807   -  INT 3

0x808-0x809   -  Timer

0x80a-0x80b   -  Serial Port Receive

0x80c-0x80d   -  Serial Port Transmit

0x80e-0x80f   -  TDM Serial Port Receive

0x810-0x811   -  TDM Serial Port Transmit

0x812-0x813   -  INT4

0x814-0x820   -  reserved

0x822-0x823   -  TRAP  - used from debugger

0x824-0x825   -  NMI

0x826-0x827   -  reserved

0x828-0x83f   -  User-defined

Please note also that the ARP (auxiliary register pointer) is

set to AR5 with executing the Interrupt vector branch! The

original ARP content is saved to the 3 MSBs in ST1 and can

here extracted and restored!

============================================================================

DSP DESIGNERS KIT ASSEMBLER

---------------------------

The DSP Designers Kit Assembler (DSK5A.EXE) is a very simple and easy

to use assembler for the TMS320C5x family of processors.  Only the most

essential features of an assembler have been incorporated to help keep

the assembler as easy to use as possible.  Every effort has been made to

keep the DSK5A as compatible as possible to TI's commercialy retailed "TMS320

Fixed Point DSP Assembly Language Tools" which offer many more features.

OVERVIEW

--------

The DSK5A assembler differs from many other assemblers in that it does not

go through a linker phase to create an output file.  Instead the DSK5A uses

special directives to assemble code to an absolute address 'on the fly'. A

label table will generate at the end of list file which can give you the

absolute address of all labels. The strength of this approach is that small

and simple programs can be created quickly and easily.  Larger executables can

also be created by chaining files together with the '.include' directive.

There is no provision for linking executables or libraries together at a later

time.

COMMAND LINE OPTIONS FOR INVOKING DSK5A

--------------------------------------

Useage: DSK5A <FILE[.ASM]> [KL] [asm "code"]

||   |

||   pre-assembles 'code' as an include

|+- generate list file FILE.LST

+--- Keep output. Ignoring ERRORS

FILES OPENED:  FILE.DSK: DSK5A Output file for use with DSK5D

FILE.TMP: Temporary object (automaticly deleted)

FILE.INC: File used when using the asm option

Example: To Assemble TEST.ASM to TEST.DSK and generate a listing file TEST.LST

C:\DSK> DSK5A TEST.ASM -L

option: -K  If this option is given DSK5A will try to generate an output

file irregardless of the outcome of any errors or warnings.

By default an ERROR corrupted file is deleted.

-L  This option generates an intermediate file containing the

unresolved opcodes.  Unresolved symbols are resolved when

the entire file has been read in.  Note that the symbols

are not stored in the intermediate object file and are

therefore not linkable.

asm  This option is a feature which allows the user to define

assembler statements from the command line.  Assembler code

in this case is terminated at both ends with a double quote.

The character immediately following the first quote is placed

in the first column.  ASM statements are written to an

include file FILE.INC overwriting the previous file using

that name.

Example:

DSK5A TEST.ASM  L  ASM"FFT .SET 256"  ASM" .entry 0a00h"

generates the file "TEST.INC" which contains

column 1

|

FFT .SET 256

.entry 0a00h

DSK5A SOURCE FILES:

-----------------

Editors:

DSK5A assembler source files can be created with almost any ASCII editor.

Legal characters are any alpha-numeric character, white space and tabs.

What the DSK5A can not work with are files created directly from word

processors that use special control characters for formatting.  Usually

these editors can generate pure ASCII text files but not by default!

Comments:

There are two ways to comment a line with the DSK5A assembler.  The first

and prefferable way is to use a semi-colon ";".  A semi-colon can be

placed anywhere in a line and everything after it is ignored.

The second way to add a comment is to use an asterisk "*".  In this case

the asterisk must always be in the first column.  If it is not, it will

be assumed to be part of an opcode and will likely generate an error.

Labels

All labels must begin in the first column and must start with a letter

or an underscore "_".  After the first letter you can then mix in numbers

as you go along.  An optional colon ":" which is usually needed in other

assemblers as a terminator is not needed.  A label can be up to 8 characters

in length.

When an opcode or directive references a label, the value of the address

where the label is located in memory is substituted.  The only exception

is the LDP opcode which loads the top nine bits of the address.

Opcodes

Opcodes must begin past the first column or after a white space or tab

character following a label.  The syntax usage of specific TMS320C5x

opcodes can be found in the TMS320C5x Users Guide.  The rules that apply

are given in the examples below.

First, it is usually neccessary to resolve ALL fields in an opcode.  If

an opcode field such as the shift field in a SACL opcode is left out,

the next field found becomes the value for that field and the next field

is not filled in.  If a field is not specified, its value is assumed

to be a null and the opcode is processed accordingly.

Second, expression analysis is NOT performed!  Things like the addition

of two values, multiplies, shifts etc... are not perfomed.  Only direct

substitution of defined variables is allowed.

EXAMPLE

.ps

LAR   AR7,RESULT

MAR   *,AR7

WRONG: SACL  *,AR7      ; becomes SACL  *,7  (AR7 used for shift)

RIGHT: SACL  *,0,AR7    ; all field specified

OK:    SACL  *          ; last two fields not specified.  This works

.ds

RESULT .word 0

This happens because the numeric value of AR7 becomes 7, which when

placed into the B opcode (in the wrong field) becomes *BR0+ in the

AR modification field.

NOTE: IMPROPERLY SPECIFIED OPCODES DO NOT ALWAYS GENERATE WARNINGS !

==========================================================================

DIRECTIVES

==========================================================================

label  .byte  value1,[value2,]...

The .byte directive places one or more bytes into consecutive memory words.

The 8 MSB's are filled with 0's.  Value can either be numeric or character

constants.  Variable substitution is NOT supported.

LAB1   .byte    -1,2,0Ah,'A'  ;where 'A' is converted to it's ASCII char

==========================================================================

label  .int   value1,[value2,]...

label  .word  value1,[value2,]...

The .int and .word directives place one or more signed 16 bit values into

memory.  Character constants are also legal.  Variable substitution is NOT

supported.

LAB1   .int     0,-1,2,0ABCDh,'B'

==========================================================================

label  .long  value1,[value2,]...

The .long directive places one or more signed 32 bit values or character

constants into memory.  The least significant word is stored first.

LAB1   .long    0,-123456,234567,01234ABCDh,'C'

==========================================================================

label  .float   value1,[value2,]...

label  .double  value1,[value2,]...

label  .efloat  value1,[value2,]...

label  .bfloat  value1,[value2,]...

label  .tfloat  value1,[value2,]...

These directives convert numeric values into various floating point

formats.  The formats are ...

.float    IEEE-784 32 bit floating point format

.double   IEEE     64 bit floating point format

.efloat   This format uses a 16 bit 2's compliment exponent and a 16 bit

2's compliment mantissa.  The mantissa's decimal point is between

bits 14 and 15 making the 1's place explicit.  That is it is not

implied as in IEEE formats where it is normaly suppressed.  In

IEEE this is suppressed to allow for greater precision since the

mantissa is always between 1.0 and 1.9999 in value.  For DSP use

an IEEE number would need to be 'unpacked' before the individual

members are operated on.  As a result speed performance would be

sacrificed.  In the .efloat, .bfloat and .tfloat formats numbers

are left in their unpacked form for greater efficiency.

.bfloat   similar to .efloat except that it has 16 bit exponent and 32 bit

mantissa.

.tfloat   similar to .efloat except that it has 32 bit exponent and 64 bit

mantissa.  NOTE: This is a proposed format for the TMS320C40 and

is only included for compatability.  It is not suitable for a 16

bit processor!

==========================================================================

.mmregs

The .mmregs will generate the symbolic names for the C50 registers and

places them in the symbol table. It is equivalent to executing

IMR   .set   4

GREG  .set   5

==========================================================================

label  .string value1,[value2,]...

The .string directive places bytes into memory in a packed form in the

order they are encountered.  If a word is not completely filled, the

trailing LSB's are filled with 0's.  Value can either be numeric, character

constant or character string.  Variable substitution is NOT supported.

LAB1   .string  -1,2,0Ah,"ABC",'D'  ;"ABC" is converted to 3 ASCII char's

==========================================================================

label  .space  value

The .space directive is used to insert value bits into the current

segment, either program or data.  Each word in the space is filled

with 0's and words not completely filled are padded with additional

zeros.

Example:

b        past      ;

.space   040h      ;fills in 4 words with 0's

past:   nop                ;

==========================================================================

var   .set    value

The .set directive is used to assign a value to a variable.  This

directive is useful in establishing values throughout the scope of

the assembled code.  For instance, .set could be used to set the

load and program entry addresses or establish values for commonly used

variables like memory mapped registers.

Example:

[add in the feature]

==========================================================================

.include   "file.ext"

.copy      "file.ext"

These directives are used to chain files together making it easier to

build large programs.  Note that because there is no link phase to

the DSK assembler that multiple definitions of labels and variables

will generate errors and warnings.  Nesting of included files of

up to 8 levels is allowed before terminating the assembler.

Example:

.include        "MMRS.ASM"    ; include a file defining the MMR's

.include        "LINK.ASM"    ; include a file defining the load

; and program entry address

.include        "VECTS.ASM"   ; include a vector table

; BEGIN YOUR CODE HERE

==========================================================================

label   .text

label   .data

These directives select that assembly output either goes to the program

or data space respectively.  The previously used address (program or data

space) is kept in memory making it easy to continue code or data after a

break.

Example

[ add in the feature ]

==========================================================================

The .end directive terminates the assembly process making any code that

follows irrelevent.  Note that an improperly placed .end directive inside

an .include file can also terminate the assembler prematurely.

==========================================================================

.ps     [value]

.ds     [value]

These directives are used to initialize the program and data load

load address's.  They can appear multiple times, making it possible

to selectively not load anything to the gaps that are produced.

Also, if the addresses overlap, it is possible to overwrite code

or data with these directives.  If [value] is ommitted, the last

address is used as in .text or .data.

==========================================================================

.entry  [value]

When this directive is encountered the value given becomes the starting

address of the program counter when the file is loaded.  If no value is

given the current program memory address as determined by the .ps or

.text section becomes the starting address.  If more than one .entry

directive is encountered in the file, the last .entry becomes the starting

address of your code.

Example

[ add in the feature ]

==========================================================================

.liston

.listoff

These directives are sometimes usefull in debugging your code.  They

are used to override the command line -L option which can turn on the

output listing.  The listing is always written to a file whose name is

derived from the original source with an extension of file.LST.

NOTE: .listoff also turns off disassembly since this feature is part

of the LIST file.

==========================================================================

.if       value

.else

.endif

These directives are used to conditionally assemble sections of code.

The .if segment is assembled if value is non-zero.  Value can either

be numeric or a preset (.set) variable.  Note that since DSK5A does

not have a link phase and is a one pass assembler, variables must be

initialized BEFORE using them!  Otherwise their value will be zero.

ALso note that expression analysis is not supported.

Example:

ELOOP   .set    1

.if     ELOOP

SELF    B       SELF

.else

NOP

.endif

==========================================================================

.qxx

.lqxx /* 32 bit Q formats */

These directives are used to generate signed 2's compliment fractional

integer and long integer values whose decimal point is displaced xx

places from the LSB.

Example:

.ds      0400h

.Q15     0.25, 0.5, 0.75

<- 15 places ->

0400     00000h  ; #0.000000000000000b

0401     02000h  ; #0.010000000000000b

0402     04000h  ; #0.100000000000000b

0403     06000h  ; #0.110000000000000b

==========================================================================

DSK OUTPUT FILES

The DSK output file supports 16 bit addresses both in program and

data space allowing direct placement of runtime code into a target.

+-- DSK characters:  K designates start of file

|                    1 designates code entry point

|                    9 designates load address

|

|          +-- DSK revision and output file name

|          |                                 +-- Checksum follows

^ |<-------------->|                         |    +-- line terminate

K_DSKA_1.01_TEST.DSK                         |    |

1FB007FB00F                                  ^    ^

9FB00B5588B55A9B55AAB55ABB55ACB55ADB55AEB55AF7AD3CF

9FB08BFFA8BFB01B67E0B60F07C379F

90400M0000M0000M123471234F

:              ^

^              |

|              +-- B designates load to program space

|                  M designates load to data space

|

+-- End of file record