DSP STARTERS KIT

"DSK"

INSTALLATION GUIDE

This file helps the user to successfully install the DSK software and hardware

to the PC.

Table of Contents:

------------------

1. WHAT IS INCLUDED IN YOU DSK KIT ?

2. CONNECTING THE DSK TO YOUR PC

3. RS232 CABLES REQUIREMENTS

4. POWER SUPPLY REQUIREMENTS

5. SETTING UP THE DSK SOFTWARE

6. DSK SPECIFICATIONS

1. WHAT IS INCLUDED IN YOUR DSK KIT ?

--------------------------------------

- DSK Installation Guide

- C5x Users Guide

- Software Diskette

- DSK Application circuit board

> TMS320C26 DSP processor

> TLC32040 Analog Interface Circuit (AIC)

> RS-232 interface chips

> AC/DC Power supply circuitry

+-------------------------------------------------------------------+

| NOTE: THE RS232 CABLE AND WALL MOUNT TRANSFORMER ARE              |

|       INSTALLATION SPECIFIC AND ARE NOT INCLUDED. REFER TO        |

|       SECTIONS 3 and 4 FOR MORE INFORMATION.                      |

+-------------------------------------------------------------------+

2. CONNECTING THE DSK TO YOUR PC

-----------------------------

1) First, turn off the power to your PC.

2) Connect an RS232 cable to either COM1 or COM2 of your PC.

3) If needed, connect an RS232 25 to 9 pin adapter.

4) Connect the DSK to the other end of the RS232 cable.

5) Connect the 9 VAC transformer to the DSK.

6) Plug the 9 VAC transformer into the wall.

7) Turn on the power to your PC

8) Install the DSK software using DSKINSTL.BAT batch bile.

Type C:\dskinstl ?  where ? is either a: or b:

3. RS232 CABLES REQUIREMENTS

-------------------------

The DSK requires a 9 pin RS232C connection to the PC host. If your comm

port has a 25 pin connector, you will need a 25 to 9 pin adapter. These

parts can be purchased from Radio Shack. The catalog numbers are listed

below:

9 -to- 9 pin RS232 link (Cat No. 26-117)

25 -to- 9 pin RS232 adapter (Cat No. 26-1388)

4. POWER SUPPLY

------------

The DSK requires a 9 VAC (>250 mA) UL class 2 transformer.  The DSK power

jack accepts 2.1mm (2.5mm in older revisions) plugs which are common to many

wall mount AC transformers.  A low current UL listed transformer is

recommended for the DSK module since they are designed to withstand most

power faults for a brief period of time.

IF YOU HAVE PROBLEMS, IMMEDIATELY UNPLUG THE TRANSFORMER FROM THE WALL

and check that your connections are correct. In the United States a

suitable transformer (9 VAC, 1.3A) can be purchased from Radio Shack.

(Cat. No. 273-1611A). If you have an older version DSK which accepts the

2.5mm plug, the catalog number is 274-1568.

5. SETTING UP THE DSK SOFTWARE

---------------------------

BACK UP THE ORIGINAL DSK DISKETTE AND KEEP IT IN A SAFE PLACE!

On a DOS command line type the following:

C:\> dskinstl a:    (installing from drive a:\)

This will exectute a batch file and creates a sub-directory named C:\DSK.

It copies all files from the original DSK diskette into the C:\DSK

sub-directory.

If you plan to develop code in another directory you may also want to

add the DSK directory to the PATH setup statement in AUTOEXEC.BAT

Edit the PATH statement in AUTOEXEC.BAT as follows....

PATH C:\DOS;C:\MOREDIRS;C:\DSK  <== APPEND ";C:\DSK" TO THE PATH SETUP

Reboot your computer. Type the following to execute the debugger/monitor

program. Make sure DSK is powered up and connected to the computer.

C:\DSK> DSK5D         <== Execute the DSK Debug/Monitor program

At this time the DSK Debugger program (DSK5D) will attempt to setup

communicate between your PC and DSK.  If the DSK5D is succesfull a screen

will appear with display boxes for the CPU registers, memory, dissasembly

and watch. If DSKD could not establish communications, it may display an

error message or continually attempt a connection.

Once DSK5D has been started, try loading the test file FUNC.DSK under the

\APPS directory.  This is done by hitting the "L" key for load, followed by

"D" for DSK in the sub-menu. Note how the top line changed to the sub-menu

when you hit the "L" key. Next type "FUNC.DSK" into the file to load field,

then hit enter. If everything worked the file should now be loaded and

ready to run.

With FUNC.DSK now loaded into the DSK module, try single stepping the

monitor by hitting the "space" bar.  As you step through the code you

should see the registers and memory being modified by the CPU.  To single

step a set number of times (without hitting "space" all the time) enter "X"

for eXecute, followed by "N" to enter the number of steps. You then enter a

value and hit enter.  To free RUN the DSK, enter "X" for execute, followed

by "G" for Go.  To halt the DSK hit the "ESC" key.

In the event of an error, the COM port, baud rate and ROM revision can

be directly specified on the command line when DSK5D is invoked.  The

DSK5D invocation help panel can be displayed from DOS by entering

C:\DSK> DSK5D ?      <= HINT: Many DOS applications use ? for online help

6. SPECIFICATIONS

--------------

Power Supply:INPUT: 9 VAC, 250 mA minimum, UL Class 2 transformer

OUTPUT: +/-5  VDC regulated, <50 mA

+/-12 VDC unregulated, <50 mA, <4V ripple (no load)

Analog In: +/-3V max

Analog Rin: 100 Kohm typ

Analog Out: +/-3V typ into 600 ohm load

RS232: Requires TX,TR,DTR and GROUND.

RS232 output is +/-5V minimum into 600 ohms

NOTE: The DC supplies are affected by external loading.  Loads in excess of

50 mA may create excessive ripple that may cause the 5VDC regulators

or RS232 driver to not operate properly.  If you plan to expand the

DSK with external memory, I/O devices etc... you will likely need

to use an external supply.